Use of Colors in REND386

Version 1.00 -- April 1992

Bernie Roehl

The REND386 package associates a 16-bit color value with each polygon in the

rendering database.  They are represented in .plg files as hex numbers.

In 256-color mode, this 16-bit value is interpreted as follows:

H R SS CCCC BBBBBBBB

The H bit is used internally to mark a polygon as "highlighted"; in the current

implementation, highlighted polygons are outlined along their edges.

Externally (i.e. in a .plg file) this same bit is used to indicate that the

color is to be "mapped"; that is, the bottom 15 bits are to be used as an

index into a color map to get the actual value to be used.

The R bit is reserved for future expansion, and should be set to zero.

The SS bits specify one of four possible surface types, as follows:

00 -- this polygon should be drawn in a fixed color, with no special

effects.  If the CCCC bits are zero, then the BBBBBBBB bits

directly specify one of the 256 available colors; if the CCCC

bits are non-zero, then they specify one of sixteen "base" colors

and the top four bits of BBBBBBBB specify which shade of that color

to use.

01 -- this polygon should be drawn with a constant shading that is

determined by the angle at which light is striking it; thus, as

the polygon moves around, its apparent brightness will change.

The CCCC bits specify one of sixteen "base" colors, and the bottom

8 bits BBBBBBBB represent the "brightness" of the color.  This

brightness value is multiplied by the cosine of the angle between

the polygon's normal vector and the vector from the polygon to the

light source; the result is used to specify an offset into the

given color's array of shades.

Note that if the CCCC value is 0, the color will always be black.

10 -- this polygon should be treated as being "metallic"; the CCCC bits

(which should be non-zero) specify one of the 16 colors, and the

top 5 bits of the BBBBBBBB value are used as an offset into a

range of shades to cycle through to give the metallic effect,

i.e. a starting offset into the color cycle.

11 -- this polygon should be treated as being "transparent"; it is just

like surface type 10, except that alternating rows of dots are used

instead of solid colors, allowing you to "see through" the polygon.

Other surface types may be defined later; hence the importance of keeping

the R bit set to zero.

The actual sequence of colors in the palette is arbitrary; the palette we're

currently using has (in general) a saturated version of each color, and an

unsaturated (watered-down) version.

Suggestions for possible new surfaces are welcomed, as are questions

concerning this specification.  Please direct any comments to

broehl@sunee.uwaterloo.ca, and I'll try to respond to them as time

permits.