Quick-Start Guide to XFree86 Setup

Joe Moss

26 August 1996

Current releases of XFree86 include several tools that can help to

automate the process of server configuration. Much of the existing

documentation, however, describes how to do the job manually, includ-

ing many technical details.  For those users with esoteric hardware or

with the desire to get their hands dirty under the hood, this is

great, but many users are using common hardware and just want to get X

up and running quickly.  This guide is for them.

1.  Before You Start

There are a few bits of information that you will need to have before

you can setup the server:

The model name of your video card

Make sure you know the exact model name of the card.  It may

help to also know the graphics chipset, RAMDAC, and clock chip

used on your card.

The amount of memory on your video card

Find out how many megabytes of RAM are on your video card.

Whether or not your card is VGA compatible

Most cards these days are VGA compatible, but for example, if

you have and older monochrome card, it might not be.

Your monitor's specifications

Specifically, you need to know the horizontal sync rate(s), and

vertical refresh rate(s).  These are important!  Consult your

monitor's manual.

The protocol used by your mouse

It will help speed up the process, if you know which protocol is

used by your mouse to communicate. Some mice are capable of

using two different protocols, although the method of switching

between them varies.

2.  What to Do - An Overview

There are three tools that can be used to set up XFree86:

o  XF86Setup

o  xf86config

o  xvidtune

XF86Setup primarily uses a graphical user interface and is the

preferred tool for initial setup, but there are a few cases where it

can't be used.  If you are using a card that is not VGA compatible,

have a fixed-frequency monitor, or are running OS/2, you'll not be

able to use XF86Setup, read about xf86config instead.  If you have

limited RAM or a slow system, you might be better off using xf86config

as well.

The xf86config program is text based only, but works for almost any

hardware combination.  If you have a fixed frequency monitor that

won't work with standard text modes, you will have to read the

necessary documentation and do the configuration manually.

To get things looking just right, you may need to use xvidtune, a

program that allows you to make adjustments to the displayed image

(e.g. make it wider, move it a little to the left, etc.).  XF86Setup

will allow you to run xvidtune at the appropriate time; if you use

xf86config, you can use xvidtune afterwards.

All of these are explained in detail in the following sections.  If

you're the type that doesn't like to read the documentation, but would

rather just try and figure your way through things, you can just type

XF86Setup now.  If you have problems, the documentation will still be

here.

Although it is possible to use XF86Setup from within X to make changes

to your existing configuration, such use is not specifically

documented here.  These instructions are primarily for those initially

setting up XFree86 on their system.

3.  Using XF86Setup

XF86Setup will first check around to make sure certain files are

installed and that you are running as root.  If a problem is found, it

will display a message and exit.  Correct the problem (e.g. install

the missing files) and run it again.

3.1.  Initial questions

If you have an existing XF86Config file, you will be asked if you

would like to use it to set the default values of various

configuration settings.  If you've already got an (at least somewhat)

working configuration you will want to do this.

If you are running on an OS which has a mouse driver in the kernel

(e.g. SCO or SVR4), you may be asked if you'd like to use it.

Once the questions (if any) are completed, you will see a message

indicating that the program is ready to switch into graphics mode.

Just press Enter.  If you don't get a graphics screen saying Welcome

to XFree86 Setup within a minute, something has probably hung, you can

try pressing Ctrl-Alt-Backspace to switch back to text mode and you'll

probably have to use xf86config instead of XF86Setup.

3.2.  Configuration areas

Once the VGA16 server is started, and once the program has finished

loading, you will see a screen with five buttons along the top and

three along the bottom.  The buttons along the top correspond to the

general categories of configuration settings.  They can be done in any

order.  Each of these areas is explained in detail below.  The bottom

row consists of the Abort, Done, and Help buttons.

Abort does as it name implies. It exits the program without saving any

changes that have been made. The one possible exception is the link to

the mouse device. Any change to that is made as soon as Apply is

selected.

Done should be selected when you've finished configuration in each of

the various categories.

The Help can be pressed at any time to get on-line help regarding the

current configuration screen.

You should start with configuring your mouse as it will make things a

lot easier to perform the configuration of other categories.

3.2.1.  Mouse

The mouse configuration screen is used to get the mouse working

properly.  There are key bindings for everything so that you can

easily configure the mouse, if it's not already working.

The screen includes a representation of a white mouse with three

buttons.  As you move your mouse it should show the pointer

coordinates on the mouse and the buttons should turn black as you

press the corresponding button on your mouse.  If that is not

happening, then your mouse is not correctly configured.

Along the top is a row of buttons corresponding to the various

possible protocols. There will also be several buttons and a couple of

sliders for other settings, a visual representation of the mouse, and

a button to apply any changes.  There may also be an entry box in

which the device can be set along with a list of possible devices.

First try moving your mouse around and see if the pointer moves

correctly.  If so, try testing that the buttons are working properly.

If those are working as desired, go ahead and go on to another

configuration area.

If the mouse pointer doesn't move at all, you need to fix either the

mouse device or the protocol (or both).  You can press 'n' followed by

a Tab, to move to the list of mouse devices and select a different

one.  Pressing 'p' will pick the next available protocol on the list.

After changing these, press 'a' to apply the changes and try again.

Repeat the process until you are getting some response from your

mouse.

If the mouse pointer or button indicators do something when you move

the mouse, but the pointer is not moving properly, you probably have

the wrong protocol selected.  Try with a different one.

Most mice these days use the Microsoft protocol, the second most

common is MouseSystems.  Some mice do both.  These dual-protocol mice

have various methods of switching between the two protocols.  Some

have a switch on the mouse itself.  Some are switched by sending a

certain signal to the mouse when opening a connection to the mouse.

These signals can be controlled by using different combinations of the

'ClearDTR' and 'ClearRTS' settings.  Other mice require a button to be

depressed when the mouse is opened (when the mouse driver first tries

to talk to it).  If your mouse uses this method, hold down the

appropriate button while selecting apply (pressing 'a').

Once the mouse pointer is moving correctly, test that all three

buttons are working properly. If your mouse only has two buttons,

select 'Emulate3Buttons' and you should be able to press both buttons

simultaneously to emulate the missing middle button.  If not all of

the buttons are working, try changing the 'ChordMiddle' setting or you

may be using a protocol that is similar to that of your mouse, but not

quite right.

3.2.2.  Keyboard

You need to specify the model and layout of your keyboard (and press

apply) if they are not already correct.  The graphical representation

of the keyboard will be updated when you choose a different model.

For non-U.S. keyboards you may wish to choose a variant from the list

(at this time there is only one available variant: nodeadkeys>).

You can also pick from the options to the right, if you wish.

3.2.3.  Card

Pick your card from the list.

If there are README files that may pertain to your card the 'Read

README file' button will then be usable (i.e. not greyed out).  Please

read them.

If your card is not in the list, or if there are any special settings

listed in the README file as required by your card, you can press the

'Detailed Setup' button to make sure that the required settings are

selected.  Otherwise, you're finished with configuring your card.

To use 'Detailed Setup': First select the appropriate server for your

card.  Then read the README file corresponding to the selected server

by pressing the 'Read README file' button (it won't do anything, if

there is no README).

Next, pick the chipset, and Ramdac of your card, if directed by the

README file.  In most cases, you don't need to select these, as the

server will detect (probe) them automatically.

The clockchip should generally be picked, if your card has one, as

these are often impossible to probe (the exception is when the

clockchip is built into one of the other chips).

Choose whatever options are appropriate (again, according to the

README).

You can also set the maximum speed of your Ramdac.  Some Ramdacs are

available with various speed ratings.  The max speed cannot be

detected by the server so it will use the speed rating of the slowest

version of the specified Ramdac, if you don't specify one.

Additionally, you can also specify the amount of RAM on your card,

though the server will usually be able to detect this.

3.2.4.  Monitor

Enter the horizontal and vertical frequency ranges that your monitor

supports in the corresponding entry boxes near the top of the screen.

You can enter specific frequencies or ranges of frequencies (separated

by hyphens).  If the monitor supports several different frequencies or

ranges, list them all, separated by commas.

If you can not find this information in you monitor's manual, pick one

of the choices from the list of common monitor capabilities.  The

program will use conservative values for each of these, so you'll get

better performance if you type in the correct values from your monitor

manual.

3.2.5.  Other

You can probably just skip this one.

3.2.6.  Completing the configuration

Once you've finished with the above, press the 'Done' button and then

the 'Okay' button which will appear. You will then be switched back to

text mode.

3.3.  Back to text mode

The program will now attempt to start the appropriate server for your

card, with all of the configuration settings you selected.  If for

some reason it is unable to start the server, you have likely selected

an improper setting and will be asked if you would like to return to

the graphical configuration screen and try again.

3.4.  The second server

This is unlikely to happen, but if when the server starts, the display

is unreadable, try pressing Ctrl-Alt-+ (using the plus on the numeric

keypad) to switch to a different video mode.

The display will show an entry box and three buttons.

The first button allows you to run xvidtune to adjust your video

modes.  One important point to keep in mind when using xvidtune is

that switching video modes with Ctrl-Alt-+ is disabled while xvidtune

is running.  You must use the 'Next' and 'Prev' buttons to switch

modes.  Because of this, you should be careful not to move the mouse

when pressing either of these.  If by some chance the mode you switch

to doesn't produce a readable display on your monitor, you can then

just press the mouse button again to move to the next (hopefully

readable) mode.

The second button causes the settings you've made to be written to the

filename given in the entry box.  After saving the settings a message

will appear indicating that it has finished. Just press the 'Okay'

button and you're done.

And the third button causes the program to exit without saving any of

the configuration settings.

3.5.  Ending text

You are returned to text mode and the program will print a

`Configuration complete.' message.  You should now have a usable

configuration file and can start the X server by whichever method you

wish (usually either the 'startx' command or via 'xdm').

4.  Running xf86config

From a text screen, run the xf86config program.  This program should

be run as root (although not absolutely necessary, it will allow

xf86config to do more of the work for you).  You can press your

interrupt character (usually Control-C or perhaps Delete), at any time

to stop the program, if you need to.  You can just start it over

again.

The xf86config program provides instructions on screen as to what you

need to do.  Following are some notes that document the various stages

in the process. They should help you get through the process quickly

and provide some documentation for those people who like to know what

they're getting themselves into, before running a program.

4.1.  The intro screen

First, xf86config begins by telling you a few things like the fact

that it can help you setup an XF86Config file or that you can do the

job yourself with an editor.  Just read what it says and press Enter

when done.

4.2.  Getting your PATH right

The program will next check that you have the directory /usr/X11R6

(the standard installation directory) on your system and tell you that

it needs to be in your PATH environment variable.

It will also check if you have the /usr/X386 directory as used by

older (pre 3.0) versions of XFree86.  If by chance you do, it will

warn you that /usr/X11R6 must be before /usr/X386 in your PATH.

If everything is okay, just press Enter and go on, otherwise press

Control-C to exit and make any necessary changes and restart

xf86config.

4.3.  Mouse setup

Pick the mouse type from the menu and enter the name of the device to

which mouse is connected, as directed.

If you are using an OS (e.g. SVR4, SCO) that has a built in mouse

driver that the Xserver could use, you'll need to edit the XF86Config

file to setup your mouse, so just pick any mouse from the list and

press enter when asked for the device.

If you don't know which protocol your mouse uses, you'll just have to

guess (the xf86config program will give you some hints as to which

might be most likely) and then see the troubleshooting section if it

doesn't work when you run the server.

4.4.  Keyboard setup

Simply answer yes to the question regarding keyboard setup.

If there is some reason you need to use the right-alt and control keys

for something else, you can enter no.

4.5.  Monitor setup

Setting up a monitor consists of entering the specifications of your

monitor and a description of the model and manufacturer.

You are first asked for the horizontal sync rate.  It is VERY

important to enter the correct value(s) from the manual. If one of the

ranges given matches the rate of your monitor, then pick it, otherwise

pick custom and enter the values from your manual.

Next is the vertical refresh rate. Again, it is VERY important that

this parameter be specified correctly.  Enter it in a manner similar

to the horizontal sync rate.

If either rate is mis-specified, it can result in damage to your

monitor.

Finally, you are asked for an "identifier", your monitor manufacturer,

and model.  You can just press enter to get through these quickly.

4.6.  Selecting your card

You are next asked if you would like to view the database of cards.

Picking your card from the list will cause the answers to the

questions in the next two sections to be filled in for you and so can

save a little time.

If your card does not appear in the list, just press q and enter to

skip on to the next step - where you'll have to answer the questions

yourself.

4.7.  Server selection

If you selected your card in the previous step, then server selection

is easy - just use the recommendation from the database.

If you have a card which uses one of the chipsets for which a specific

server exists (Mach8, Mach32, Mach64, AGX/XGA, 8514/A, S3, ET4000/W32,

I128, P9000) you'll want to pick the accel option.

Otherwise you'll probably want to use the SVGA server.

Next, answer yes when the program asks if you want it to set the

symbolic link for you.  If you picked the accel option, you'll also

need to indicate which particular accelerated server to link to.

4.8.  Screen/Video configuration

Pick the appropriate option from the list to indicate the amount of

memory on your video card.

Then you are asked to provide and identifier, the manufacturer, and

the model of your card.  You can just press enter to skip through

these, if you wish.

Next, the program will ask for the type of RAMDAC and Clockchip on

your card.  If your card was in the database, you should just to tell

it to use the values from the database.

If you don't have one of the listed RAMDACs or Clockchips on your

card, just press enter when asked what type you have.  If you do not

have a programmable clock chip, the program will next attempt to probe

to find out what clock rates are supported by your clock chip.

4.9.  Mode Selection

Now you get to tell the program which video modes you would like to be

able to run.

The program will show you the common modes that should work with your

card (some might not work with your monitor, but if you've correctly

specified the monitor's sync rates, the X server will just ignore them

when it runs).

You could just accept the settings as they are given, but you'll

probably wish to reverse the order.  For example, if you have a card

with 1 Meg RAM, it will list the modes

"640x480" "800x600" "1024x768" for 8bpp

Select 1 to change the settings for 8bpp and the type 432 to select

the reverse order.

When you've select the modes, in the order you wish, select option 4

to continue.

4.10.  Creating the XF86Config file

The program will now ask if you would like to write the configuration

settings you've selected to the file XF86Config. Answer yes.

4.11.  Some final notes

Lastly, the program tells you that it's finished its part of this

process and counsels you to check the file before using it.  The next

section covers the changes that are most likely to be needed.

5.  Fixing the XF86Config file

Use an editor to look at the XF86Config file. Here are some things

that may need to be changed:

o  If you are running an operating system which has built-in mouse

support, you'll want to change the Pointer section.

Specifically, you should set the Protocol to OSMouse (SCO) or

Xqueue (SVR4, some SVR3) and you should remove the Device line.

o  If you are running a system with the Xqueue event driver and

would like to use it, change the Protocol setting in the

Keyboard section to Xqueue.

Once you are satisfied that the configuration is correct, copy the

XF86Config file to /usr/X11R6/lib/X11 and run the 'startx' command.

You should now have a running X server.  If it's running but the

display doesn't look as good as you think it should (i.e.  it doesn't

fill the whole screen, it's off-center, it's wrapping around on one

side, etc.) see the section on xvidtune.  If there is some other

problem, see the troubleshooting section.

6.  Running xvidtune

If you need to make adjustments to the video display, xvidtune is the

tool to use.

Simply enter the command xvidtune from a shell prompt within an xterm.

Read the warning and click on OK.  Next click on the Auto button.

Now click on whatever combination of Up/Down/Left/Right

Shorter/Taller/Wider/Narrower is need to adjust the display to your

liking.

If you are using a recent S3-based card there will be some extra

buttons and entries at the bottom (InvertVCLK, EarlySC, and Blank

Delays). These can help solve problems of the display wrapping around

a few pixels.

Once the display has been adjusted properly, press the show button to

printout the correct ModeLine to put in the XF86Config to make the

server always use the current display settings.  To aid in copying

this information to your XF86Config file, the modeline is also made

the current selection allowing you to just paste it into your editor.

If you would like to adjust your other modes, you can click on the

Next and Prev buttons to switch modes.

When you are through using xvidtune simply press on the Quit button.

7.  Troubleshooting

Since you're reading this, something must not have gone the way you

had hoped (or else you just really enjoy reading).

Below are listed some common problems that may occur during

configuration and some hints for solving them.  However, there are

just too many different combinations of hardware and software

configurations, and, well, just too many things that can go wrong, for

this document and the tools it documents, to cover every case.

If after trying the steps in the previous sections and checking the

hints in this section, you still are unable to get your system

working, you'll have to read the full documentation.  Read the README

file for your card and OS (if they exist), the XFree86 Configuration

Guide (README.Config), and the XF86Config man page.

You should also look at the XFree86 FAQ <http://www.XFree86.org/FAQ>

for more up-to-date information, especially if you are trying to

configure a fairly new card.

If all else fails, you can try posting a message to

comp.windows.x.i386unix or comp.os.linux.x or send email to

XFree86@XFree86.org.

7.1.  The mouse doesn't move correctly, it stays in one area of the

screen

You've selected the wrong protocol for your mouse. Try a different

one.

7.2.  The server doesn't start, it says the mouse is busy.

Well, it's probably right.  This most often happens on Linux systems

that have gpm running.  Kill the gpm process and try startx again.

7.3.  The middle button doesn't work.

There's no easy answer to this one.  It's a lot of trial and error.

You need to make sure you're running the right protocol for your

mouse.

Many three button mice are "dual protocol" which means that they have

both a 2-button and 3-button mode. The way to get the mouse to switch

into 3-button mode (which usually then uses MouseSystems protocol)

varies between different models.

You may need to slide a switch on the mouse or hold down the middle

button when starting the server.  Other methods of switching modes can

be done by the server, you just have to find the right combination of

settings for your mouse.  See the Pointer section of the XF86Config

man page for a complete list of settings.

7.4.  The display is shifted to the left/right/top/bottom

See the section on xvidtune.

7.5.  I don't appear to have xf86config or xvidtune on my system

Hmmm. Three possibilities:

1. You have a version of XFree86 that is older than 3.1.2.  If this

is the case then you probably aren't reading this document

either, because it wasn't included in 3.1.2 and earlier

releases.  Please upgrade to the latest version.

2. Your PATH is not set correctly. Make sure it includes the bin

directory for the XFree86 binaries (usually, /usr/X11R6/bin

3. You don't have a complete installation of XFree86.  Go back to

wherever you got XFree86 and get the missing pieces.

Generated from XFree86: xc/programs/Xserver/hw/xfree86/doc/sgml/QStart.sgml,v 3.2 1996/08/27 03:21:12 dawes Exp $

$XFree86: xc/programs/Xserver/hw/xfree86/doc/QuickStart.doc,v 3.3 1996/08/27 03:23:59 dawes Exp $