DU50 - Formatter of discs for the Spectrum +3         John Elliott, Dec. 1992.

This file is a direct port of the Amstrad CP/M program DU49, only the formats

are different. Because it is a port, there are certain features which worked

under CP/M but don't seem to work under +3DOS.

The main one of these is "Select disc type". It *should* fix the XDPB of a

drive, so that only discs of that format can be used in it. This would allow

198k discs rather than 192k as the maximum size. This works fine in CP/M, but

+3DOS seems to have trouble with it.

Formats available are:

"Amstrad Autoselect" - not of use when formatting. When "selecting", it should

release a drive so that discs of any format can be read in it.

6128 System and Data formats: 169k and 178k respectively. 64 dir. entries.

Large directory:             +3DOS disc with 3 times as many directory entries.

Normal Spectrum:             As made by FORMAT.

Normal XCF2:                 192k; 92 dir. entries. Ten sectors per track.

Large directory XCF2:        189k; 192 dir. entries.

XCF2 with no system track:   198k. This format is not detected correctly unless

the format is fixed with option 1 - and option 1

doesn't seem to work properly.

PCW Drive B:                 Only for double-density (eg 3.5") disc drives. 706k

XCF2DD:                      Only for DD drives. 784k. 256 entries.

XCF2DD no sys:               This is another non-auto-detect format. 792k.

XCF2DD no sys small dir:     And this is another. Only 128 dir. entries.

The formats which don't autodetect can be read in CP/M with this program's

counterpart, "DU49A" (which is DU49 with these formats installed) or DU49

itself if your chosen format exists in it.

DU50 is the +3DOS loader; DU50.HED is the machine code part. Copy both onto

a disc and type LOAD "DU50".

John Elliott - m93jce@ecs.ox.ac.uk