+-------------------+

|    AutoCAD-PCB    |

+-------------------+

Software Accessories For Electronic Design Using AutoCAD(tm).

["AutoCAD" is a registered trademark of Autodesk, Inc.]

+==============================================================================+

| SUMMARY:                                                                     |

| This package contains the software "add-ins" to make AutoCAD capable of doing|

| manually routed printed circuit board designs.  It has "script" files that do|

| the initial setup for these kind of drawings, menu files that create screen  |

| menus optimized for the commands most frequently used in electronic design,  |

| and a collection of pre-drawn blocks that can be inserted into drawings to   |

| speed design work.                                                           |

+==============================================================================+

Shareware Stuff . . .

These tools were developed by Paul Marxhausen of the University of Neb-

raska-Lincoln Engineering Electronic Shop to provide engineering students

who have access to AutoCAD-equipped PCs a method of doing electronic design.

They may NOT be sold or distributed for any form of compensation.

The user is encouraged to use these tools and modify them however they wish;

however, if you wish to distribute and share modifications of these files,

please do so under a name other than AutoCAD-PCB and in a different archive

file, if you are sending them to a BBS or Internet archive.  We would like to

keep the integrity of the file list above intact.  Our support for these tools

begins and ends with this free distribution, released in December 1992; ad-

ditional enhancements or releases are not planned at this time.

INTRODUCTION

Computer Aided Design is a very powerful way to create printed circuit boards,

especially when one has the use of professional-level software packages that

"capture" computer-drawn schematics, or have auto-routing capabilities.

However, use of CAD for PC boards involves the obstacles of  A) the often

substantial amount of time invested in learning the software; B) the intricacies

of getting from a bunch of bytes on a computer to a finished printed circuit

board; and  C) the substantial obstacle of COST.  Good quality PCB CAD programs

can cost upwards of a thousand dollars.  Inexpensive CAD programs can be

inflexible or inadequate.  AutoCAD-PCB was created to afford persons with access

to AutoCAD a way to do PCB CAD work quickly, inexpensively, and effectively.

NOTE TO NEW AUTOCAD USERS:

While these tools make it faster and easier to set up and edit electronic

drawings, it is assumed that the user has a basic working knowledge of AutoCAD

commands and procedures.  For example, this manual doesn't spell out how you

select an object with the pointer or a window.  If you are new to AutoCAD, be

advised that while it has menu features and on-line help (HELP command), it is,

like many fully-featured CAD packages, a very complex, elaborate, and sometimes

confusing piece of software.  However, it is also very powerful: if there's

anything you want to do in a drawing, AutoCAD has a way to do it.  If you are

serious about this work, you'll want to have access to the AutoCAD manuals or to

one of the many books available on the use of AutoCAD in local bookstores.

NOTE TO ALL USERS:

Read ALL of this documentation, start to finish - it's not that long.  It's

important to understand what these tools do with AutoCAD, or you'll end up doing

a lot of uneccessary swearing and hair-pulling, which is all too common when

using AutoCAD in the first place.  Especially pay attention to what the custom

menus do, because some of the menu items are not what they seem (see the PTRACE

menu, for example.)

THE TOOLS

THE SCRIPTS:

An AutoCAD script is a file of commands that AutoCAD can execute in "batch"

mode.  Scripts can be executed either by using the SCRIPT command while running

the AutoCAD drawing editor, or by giving a script name when you run AutoCAD from

the MS-DOS command line.

Example:

C:>ACAD MYFILE PCB  <return>

This will start AutoCAD and create the new drawing MYFILE.DWG. The script file

PCB.SCR will set up AutoCAD for certain predefined parameters.  NOTE, however,

that you actually only have to run the setup script when creating a NEW drawing,

because after you save your new drawing, all the setup information, menus, etc.

will be saved with it and all you will have to do in the future is load your

drawing with the normal ACAD MYFILE command line.

[If you're already in the middle of running AutoCAD and are editing a drawing,

you can do the same setup using the AutoCAD SCRIPT command.  However, use the

PCB2.SCR file if you do so, or it'll mess up trying to create a new drawing.

(ex: SCRIPT PCB2)]

If you want a handy way to create new drawings, use the batch file called

PCB.BAT.  It's just one line:

ACAD %1 PCB

Now, if you type  PCB  MYFILE  it'll do the same thing shown above.

The PCB Script will do several things: set up the screen units, size, grid,

axis, and snap units; define several drawing layers; and load a customized menu.

Screen and Units

The screen units used with AutoCAD-PCB are mils, or thousanths of an inch.  (One

mil = .001 inches).  This is common usage in printed circuit board design work.

It's easier to work with AutoCAD when you can tell the program that you want a

trace that's 13 mils wide instead of .013 inches, or to set the snap units to 25

mils instead of .025 inches, etc. The limits of the drawing are set to 11,000

mils by 8,500 mils, or 11 by 8.5 inches, to correspond to the standard letter

sized sheet of paper that most drawings will be printed or plotted upon.

(Please be sure to read the section on "Printing, Plotting and Production"

before you try to get a hard copy of your drawing, or you may be faced with the

task of trying to print out a drawing that AutoCAD thinks is almost a thousand

feet long, actual size.)  Note that you can change your limits or draw outside

of them if you wish.

The SNAP is like an invisible grid whose size the user can define.  When drawing

lines or selecting points on the drawing, the cursor will "snap" to these points

only if SNAP is on.  This makes it easy to line up objects. The SNAP defaults to

25 mils, and can be toggled between that setting and a 100 mil snap with the

SNAP submenu.  These snap settings are useful in PC board work because most

standard pin spacings are .1 inches, or 100 mils.  Thus, using a 100 mil snap

makes it easier to place parts on the board in proper alignment, while the 25

mil snap gives finer control for routing traces while still making it easy to

line up with objects on the 100 mil grid.

The screen grid defaults to 500 mils, or a half inch, to give the user a rough

idea of where he is on his drawing.  The axes around the edge of the drawing are

spaced at 100 mils to make it easy to line up parts.

The drawing defaults to FILL OFF, which means that thick things like traces and

pads are drawn as outlines, not areas completely filled with color.  This is for

reasons of speed: drawing a lot of complex, filled entities really can make your

computer groan.  If your drawing is small or you have a fast '486 computer

or you just can't live without the filled lines, enter FILL ON.  However, any

non-filled structures you happen to have on screen won't fill unless you also

type REGEN to regenerate the drawing.

+=============================================================================+

| IMPORTANT: AutoCAD-PCB assumes that you are drawing the circuit board from a|

| TOP VIEW, looking down at the component side of the board.  In effect, as   |

| you draw the traces you are doing so as if you had a transparent board.     |

| This is because integrated circuit data books always give their chip pin-   |

| outs as a top view, and it's easier to route your traces in a top-view mode.|

| However, when it comes time to plot or print the traces to make a circuit   |

| board negative or similar, it will be necessary to MIRROR this view, which  |

| is easily done in AutoCAD.  For more details, consult the Printing and      |

| Plotting section below.                                                     |

+=============================================================================+

While these default settings are useful, you may want something different.  Any

of these settings can be changed at any time with the appropriate commands.  If

you want to change the defaults for these or any other settings, you could edit

the PCB.SCR file with any text editor.  However, if you do want to make changes

in the start-up script, we would prefer that you copy PCB.SCR to another file

name and make your changes on that file for your own use, so that there doesn't

end up being a whole lot of different variations all named PCB.SCR associated

with this software package.

THE LAYERS

AutoCAD works with "layers"; it's very similar to having a stack of

transparencies that you can switch between and draw on individually.  This

allows the user to make certain structures a different color, or hide them

from view so that only certain structures are dealt with, or many other

useful things.  Similar to standard electronic CAD practice, AutoCAD-PCB makes

up these distinct layers:

LAYER   COLOR   FUNCTION

---------------------------

PADS    CYAN    The solder pads associated with all component leads.

BOTTOM  BLUE    The traces on the bottom side of the circuit board

TOP     RED     Traces or jumpers on the top side of the circuit board.

PARTS   MAGENTA The physical outline of the electronic components.

LABELS  GREEN   Any labels or text associated with parts or points on board.

OUTLINE WHITE   The physical edges of the PC board, and any mounting holes.

Commercial PCB CAD programs often have a good many more layers than this,

but for most single or double sided boards, these will suffice.  You can always

create more layers if you wish.  The LAYERS command in the custom PCB menu

makes it easy to switch between these layers and turn them off and on.

IMPORTANT: as will be discussed at length below, just because you are using

a certain layer doesn't mean everything you create is going to be on that

layer.  When you insert the predefined shapes, their component parts will

appear on the layers for which they were created.

THE BLOCKS

"Blocks" in AutoCAD are just pieces of drawings that you can insert wherever

you need them.  You can make anything that you draw into a block with the

BLOCK command and put it into your drawing with the INSERT command.  If

AutoCAD doesn't find a block you've asked to insert already defined inside of

your drawing, it will go out to your default disk drive and directory and look

for a .DWG file of the same name, and use it for the block.  If you have all the

AutoCAD-PCB .DWG files in your directory, you'll have access to them as

predefined blocks.  The blocks are the following:

BLOCK           DESCRIPTION

---------------------------------------------------------------------------

OP60            Oval Pad, 60 mil.  Used where traces will be run between pads.

SP60            Rectangle Pad, 60 mil.  Designates pin 1 on the DIP blocks.

RP60            Round Pad, 60 mil.

RP70            Round Pad, 70 mil.

RP80            Round Pad, 80 mil.

RP90            Round Pad, 90 mil.

RP100           Round Pad, 100 mil.

RP120           Round Pad, 120 mil.

8DIP            8 pin Dual Inline Pin integrated circuit socket.

14DIP           14 pin DIP socket.

16DIP           16 pin DIP socket.

20DIP           20 pin DIP socket.

24DIP           24 pin DIP socket.

28DIP           28 pin DIP socket.

40DIP           40 pin DIP socket.

R1_4W           1/4 Watt Resistor.

R1_2W           1/2 Watt Resistor.

R1W             1 Watt Resistor.

R2W             2 Watt Resistor.

DISCAP1         Disc capacitor.

DISCAP2         Disc capacitor.

RADCAP1         Radial lead capacitor.

RADCAP2         Radial lead capacitor.

RADCAP3         Radial lead capacitor.

RADCAP4         Radial lead capacitor.

RADCAP5         Radial lead capacitor.

AXCAP1          Axial lead capacitor.

AXCAP2          Axial lead capacitor.

AXCAP3          Axial lead capacitor.

HEADER8         8 pin Dual In Line (100 mil spacing) header socket.

HEADER10        10 pin header socket.

HEADER14        14 pin header socket.

HEADER16        16 pin header socket.

HEADER20        20 pin header socket.

HEADER26        26 pin header socket.

HEADER34        34 pin header socket.

HEADER40        40 pin header socket.

HEADER50        50 pin header socket.

HEADER60        60 pin header socket.

DB9             DB9 type socket.

DB25            DB25 type socket.

DB37            DB37 type socket.

DIODE1 - DIODE2         Diodes.

XTAL                    Crystal.

When you INSERT these, you'll see that these blocks will have solder pads on

the PADS layers, component outline on the PARTS layer, etc. regardless of

which layer you are currently using.

The discrete parts, like the resistors and capacitors, aren't strictly

necessary: you can simply use a pair of pads anywhere you need such a part.

The handful of discrete parts included are simply for convenience. There is

such a wide range of sizes for devices such as electrolytic capacitors that

you will probably have to do some measuring of your own and use discrete solder

pads for such parts.

Please note that the names for these parts in the INSERT submenus are different

than these file names.  (Example: Menu displays "Res 1/4W" instead of "R1_4W").

This is to make the menu more user-friendly and easy to understand.  However,

you should be aware of these differing names if you are entering the INSERT

command from the command line instead of using the INSERT submenus to pick out

blocks.  Thus, if you type INSERT <Ret> and the program prompts you for a block

name, you would enter "R1_4W" to get your quarter-watt resistor.

Wait a Minute - Where Did My Part Go?

There's only one tricky thing about using these blocks: AutoCAD treats them as a

SINGLE UNIT, not as a collection of pads and lines, etc.  This is great when you

want to move a block around; you do a MOVE and point to the block and the whole

thing moves around together.  This is terrible when you just want to change or

move one little part of that block.  For example, if you wanted to erase a

single solder pad on the block IC40, you'd enter ERASE and point to the pad, and

the ENTIRE BLOCK will select.  Continue, and the whole thing will erase.  How do

you get around this?

Two ways: first, you can enter INSERT from the command line. When you are

prompted for the block name, put an asterisk (*) before the name of the block,

for example, " *DIP40 " instead of " DIP40 ".  When you execute an INSERT this

way, it puts the block into the drawing as a collection of individual lines and

shapes, instead of as a single unbreakable block.  You can then erase, move,

copy, or otherwise edit individual parts of the block.  HOWEVER, you can't go

back after you've done this; if you want to move the entire block now, you'll

have to do a MOVE and then WINDOW, then bracket the whole thing with a window.

The second way to edit parts of a block is to use the EXPLODE command.  This is

a little better, because you can go ahead and use the menu or command line to

INSERT your block in the usual way, but if you find you have to go back to make

a change on a part of it, just type EXPLODE and point to the part.  It'll end up

looking exactly the same, but it's no longer a single unit.

THE MENUS

The custom menu (PCB.MNU/PCB.MNX) loaded by the PCB script simply attempts to

make some of the commands commonly used when drawing PC boards easier to access

by way of the menu on the right hand-side of the screen.  (All references to

menus in this document refer to this menu, NOT to the drop-down menus at the top

of the screen.)  Additionally, some of the predefined blocks are more easily

accessed by this custom menu.  However, it is possible to use AutoCAD and these

software add-ins entirely from the command line.

Notice that some of the menu items don't actually perform the command when

you select them; instead, they produce a SUBMENU of commands.  For example,

selecting ZOOM will get you a list that includes "In_2X", "Extents", "Window",

and a number of other ZOOMing options.  Selecting one of these will actually

execute the desired command, with the option.

Some of the Submenus don't cover up all of the main menu, and you can continue

to access commands on the main menu that are visible; however, you can end up

with yet another submenu laying over the top of the last one, and things can get

confusing.  SO, the best thing to do is to exit a submenu when you're done with

it by selecting the -BACK- command at the bottom of the submenu.  This gets

especially important when a submenu has a sub-submenu, as with the INSERT

command where there are several classifications of blocks that you can insert.

MENU TOUR

Let's just look at all of the custom menu items, in the order they appear on the

menu.  Some are simple; some have multiple layers of submenus.  We'll look at

them all.  READ ALL OF THIS: there are things that are discussed later in this

manual that assume you know everything here.

- CANCEL

The *CANCEL* menu item that is always at the top of the menus is just a way to

execute a Control-C with the menu instead of the keyboard.

- PTRACE

The traces created by the menu item labeled "PTRACE" are NOT drawn with

AutoCADs TRACE command.  Instead, when you choose a trace from the PTRACE

submenu, you'll be drawing it with the PLINE command.  Why is that, when AutoCAD

has a perfectly good TRACE command that lets you draw traces of any width?  Two

reason: first,PLINE is a lot more flexible.  You can do some amazing things

like make the width of the trace vary continuously as you go, or make it go in

curves with the ARC option, or many other things.  The second reason has to do

with the fact that when you draw something with the AutoCAD TRACE command, you

end up with something that is made up of a bunch of independent little sections.

Try to MOVE, ERASE, or COPY a TRACE, and you have to select every little section

of it wherever it changes direction.  In contrast, a trace made as a PLINE is

treated as one undivided unit from start to finish, no matter how many twists

and turns it takes.  The first time you have to erase a long, elaborate trace to

redo it,you'll appreciate being able to just do an ERASE and point to any point

on the whole trace, instead of having to point to each of many little sections.

- - DrawPowr, DrawWide, DrawMed, DrawThin

The PTRACE menu can give you traces of any width you'd like.  (By the way,

don't try to just use the LINE command for signal traces, hoping that your

plotter will make wide enough lines.  Not a reliable method at all.)  The custom

menu gives you a choice of four: Thin (13 mil), Medium (30 mil), Wide (60 mil),

and Power (100 mil).  My general practice is to use traces that are plenty wide

when I can; it means you have less copper to etch off, less chance of a minor

flaw in the drawing or negative making a break in the trace, and you have less

lead resistance.  I use the Medium width where there's room for it.

- - Chamfer

To "chamfer" something means to cut little 45 degree angle edges on places where

lines meet at 90 degrees.  This is pretty common on printed circuit boards.

With the chamfer command at your disposal, you can go ahead and just run all

your traces with right-angle corners, then go back and angle the corners later.

The chamfer distance, which is how far back from the 90 degree corner the angle

starts, defaults to 50 mils.  You can change this by entering the CHAMFER

command from the command line and specifying a different distance.

- - Edit

Choosing Edit on the submenu gets you a LOT of other subcommands.  And choosing

VertEdit [Vertex Edit] on THAT subsubmenu gets you a lot more items, which can

get pretty confusing; but these menus can let you do powerful things like move

or add corners, etc.  Here's where you'd be wise to sit down with the AutoCAD

manual and/or the AutoCAD HELP command; read all you can about the PLINE command

and the PEDIT command, and you'll understand what these menu items are

attempting to do.  Again, you can do a PEDIT from the command line too; the

menus are for speed and convenience, and once you know what the commands are

about, they will speed your work.  This is one case where using the AutoCAD

manuals is HIGHLY RECOMMENDED - the PEDIT commands are very powerful and useful,

but it's easy to enter the wrong thing at the wrong time.  'Nuff said.

- LAYER

This menu lets you switch between the various layers of the drawing and turn

them on and off.  See the section on LAYERS for an explanation of what layers

ACAD-PCB defines and how they are used.

- INSERT

Insert lets you insert the predefined blocks or any other blocks you may have

defined.  The predefined AutoCAD-PCB blocks are found under one of several

submenus: Caps (capacitors), DIPs (IC sockets), Pads (solder pads), Resistor

(guess), Connects (connectors), and Misc (odds and ends).

- LINE

Executes AutoCAD LINE command without any bells or whistles.

- COPY

Executes AutoCAD COPY command.

- ERASE

Executes AutoCAD ERASE command.

- MOVE

Executes AutoCAD MOVE command.

- ZOOM

Executes AutoCAD ZOOM command, but also provides some ZOOM options as a submenu.

- PAN

Executes AutoCAD PAN command.

- TEXT

Executes AutoCAD TEXT command.

- SNAP

Gives you a submenu to set snap to fine (25 mil), coarse (100 mil),

user-specified, or turn it off or on.

- HELP

Executes AutoCAD HELP command.

- SAVE

Executes AutoCAD SAVE command.

- SAVEEXIT

Executes AutoCAD END command.

- QUITEXIT

Executes AutoCAD QUIT command.

More Menu Customizing

Got some ideas for improvements of your own?  Want to add a couple of commands?

The custom menu is created with the file PCB.MNU.  Take a look at this file with

a text editor and if you're at all handy with programming you'll catch on

quickly how AutoCADs menus are built.  Again, the AutoCAD manuals describe the

format of this kind of file in detail.  Once a .MNU file has been created, it is

compiled into the .MNX file that AutoCAD actually uses at run time by using the

MENU command while running the AutoCAD editor.  [The author of these tools used

the SHELL command to change the PCB.MNU file while still running AutoCAD, then

used MENU to load/compile the menu to see instantly how the changes worked.]

DRAWING A BOARD

How Many Sides?

So much for tools - how do you draw a board?  First, you'd better decide on how

many sides you're going to be using.  To give you some idea of what's involved

with this decision, and to show you how the "pros" do it, get yourself just

about any commercially manufactured printed circuit board, such as an expansion

card from a personal computer or similar.  Take a look at how the chips are laid

out and the traces are run.  People making boards for a living have a big stake

in giving their boards a tight, efficient layout.

However, people making boards for a living use something you may not want to get

involved with, and that's called "multi-layered boards."  Time was, PC boards

only had traces on the bottom side.  And in fact, not only can you do a lot of

projects with single sided boards, but they are far and away the easiest and

cheapest kind of PC board to produce . . . from the standpoint of exposing,

developing, and etching them.  But even on a simple design, you're going to find

traces that you just can't seem to get from here to there without crossing

something, and you'll start putting jumper wires on the top of the board.  From

there, it's only a short jump to wanting to do a double-sided board.

The tradeoff?  Well, putting traces both top and bottom can cut your design time

considerably, because you spend less time trying to snake leads around some

torturous path from here to there without crossing another lead.  You can always

just run your problem trace on the other side of the board.  You'll see a lot of

this on commercial boards: traces that stop and switch over to the other side

of the board two, three, or four times before they get to where they're going.

Being able to do this makes it easier for the designer, but tougher on whoever

is fabricating your finished board.  In particular, if you're etching your own

board, you probably won't be able to make "plated-through holes", where there is

metal between pads on the bottom and top.  Without such plating, you HAVE to

solder leads on BOTH sides of the board if you need the signal to get to both

sides: this can be very difficult to do with many integrated circuit sockets.

At the UN-L E.E. Shop, we try to stay with single-sided designs unless we have a

large number (30 or more) of jumpers going on the top side.

Commercial boards don't stop at double-sided any more, either: many of them,

especially computer boards, actually have additional layers INSIDE the board.

(The board is made by laminating very thin circuit board layers into a single

piece.)  Usually, these internal layers will be used to make a ground plane that

takes ground to all appropriate pins; similarly, there will often be a layer

dedicated to getting 5 volts to all the chips.  This makes a board designers job

even easier, but it makes production by anything less than a professional

fabricator pretty much impossible.

OK, OK, But How Do I Draw The Board?

No sweat.  Use the menus or command line to get to the OUTLINE layer, draw the

outline of your board; go to the PADS layers and use the predefined blocks to

drop your IC sockets and other component pads where you want them; then go to

the BOTTOM layer and use the PTRACE menu to hook up wires from pad to pad.

Voila, you're done!  Right?

Well, almost.  Actually it's not much more than just that.  It's just that it

can take a lot of time trying to get the darn traces from Point X to Point Y

without crossing each other.  And how do you decide where to put your chips?

And what about running traces between pins?  And on, and on . . .

As far as chip placement goes, you have to have a good idea of how your circuit

is wired.  If you don't have at least a scribbled schematic with the pin numbers

of your chips labeled on it, you're really going to be groping in the dark.

Really, component placement is just a mixture of good common sense, a good eye

for topology, and a computer video game player's passion for solving problems.

(Being good with mazes helps, too!)

Try starting with obvious things: for example, most 28 pin DIP memory devices

have similar pin-outs that facilitate having them sit all in a row.  You're

probably going to want to have them as close as possible to the CPU, if this is

a microprocessor board.  What side of the CPU does the data buss come out of?

You're trying to make the simplest, shortest, and most direct connection between

devices that have to be connected.  You're also going to want to keep your

analog and digital sections of your circuit seperate from each other when you

can, to keep digital noise out of your analog circuits.

Aside from that, you can make your own rules about the layout.  Sometimes you

have to "break the rules" in order to get everything routed.  For example, it's

a common sense practice to have most of your chips oriented the same way, with

the notched ends all pointed the same direction - it's logical and looks good;

but I've had to put the chips in "backwards" before just because that was the

only way to get the traces routed in a straightforward way.

Which Traces First?

It's a good idea to take care of some connections before others.  For example,

on our hypothetical microprocessor board, you're going to have a keen interest

in where the data buss lines have to go, because they have to go a lot of places

and they all should go there together (unless you want to route each one

individually.)  More obviously, how are you going to get power and ground to all

of your chips?

This is a point where the multi-layer people have it good.  They can forget

about worrying about power and ground because they have entirely different

layers to deal with those.  But when you're struggling to route a single-sided

board, you'll find that those darn power leads seem to be in the way every time

you're trying to place a signal trace someplace.  Yet most design guides say to

run power and ground first!  What do you do?

I've taken to running nice fat power and ground traces more or less around the

outer edges of the board, then using jumpers on the top to bring them over to

the chips.  This eliminates some of this difficulty.  And you will find that

after all is said and done and drawn, that you'll be able to make some of the

power and ground buss connections on the bottom with traces.  It's just that I

feel that if I'm going to have to use a few jumpers here and there anyway, I'd

rather use them on power and ground to simplify my life, instead of having to

jumper, for example, all eight data buss leads over a power supply trace.

Many times you have to work in tight spaces, ESPECIALLY if you have to run a

trace between two pads that are 100 mils apart.  At that point, you have to go

to a Fine trace, or you can end up with traces and pads that look like they have

space between them when you're zoomed in close on the screen, but which will end

up touching when you print or plot the thing out, or have a negative made of

your drawing.  When you are trying to run leads between pads, it's almost

certain that you'll have to use the 60 mil width pads (OP60, SP60, RP60) and the

13 mil Fine trace. [I've seen professional CAD packages that claim to be able to

put two or even three traces BETWEEN two pads that are on 100 mil centers; fat

chance.  You'd have to have awfully skinny traces and mighty tiny pads, which

may be possible with professional boards but is an invitation to frustration and

no-functionality with "amateuer" boards.  Even if you manage to produce the

extremely high-resolution output needed, you'll have traces that break easily

and peel up when you solder to them.]

For running power and ground leads, using wider traces is advisable.  I often

use the Wide trace for this, while the Power width is so wide that I generally

reserve it for use in those big fat power and ground busses that I run around

the periphery of the board.  Again, this is all pretty much common sense stuff.

Not enough width choices on the menu?  Enter TRACE from the command line and it

will ask you for the width first, defaulting to the width of the last trace if

you just hit <Return>.  Think you used the wrong size?  The "Edit" item under

the PTRACE menu (which executes a PEDIT command) will let you change the width

of an entire trace after the fact, which is nice.

How About Some Extra Caps?

If you weren't already aware of it, it's a standard practice with circuit boards

using logic chips to place a small (.01 to .1) "despiking" capacitor from power

to ground of every chip, AT THE CHIP.  Even with a nice solid five volt supply

attached to your board, power often has a long way to go to the chips, sometimes

down some very skinny traces.  The resulting added lead resistance means that

"spikes" or fluctuations can occur on the power supply right at the chip as

devices switch or change their current demand.  This can lead to all sorts of

really, really frustrating and untraceable malfunctions.  Do yourself a favor;

despike everything you can.  In addition, I will often add one small (10 to 100

microfarad) electrolytic cap across the power supply to ground on the board,

often at the point that is farthest from where power connects to the board.

This helps to keep the power supply solid and smooth.

Making Changes

Unless you're making a very simple circuit on a very large board the odds are

that at some point in your drawing, you're going to have to make a lot of major

changes.  The chip layout you chose just doesn't get enough leads close enough

to where they have to be; or the circuit is too large to fit on the board you've

chosen; or any other of a number of things can happen.  In any case, this is

where using CAD to draw the board absolutely beats anything else hands down.

Want to move a whole chunk of the circuit to another spot?  Enter MOVE and

select the parts and just drag them where you want them.  Have a part of the

circuit that is duplicated several times?  You can use the COPY command to make

multiple copies of anything and drop them wherever you want them.

The only bad thing is that if you do start moving things around, you're going to

end up having to erase a lot of traces and redraw them.  Unlike real electronic

CAD packages, the traces you draw with AutoCAD won't "rubber band", or stretch

and contract to stay connected to the pads.  You've just got to draw them over.

However, you can do some moving of the trace corners with the PTRACE menus

"Edit" submenu (really the PEDIT command.)

There isn't enough space here to explain all of the many things AutoCAD can do

to modify your drawing, like ROTATE parts, CHAMFER traces, SCALE structures up

and down, etc.  Again, make liberal use of the HELP command and the AutoCAD

manuals and you'll find a wealth of commands that can do just about anything you

have in mind.

OOPS . . .

And above all, check out the OOPS and UNDO commands, which can back you out of

even the most catastrophic mistakes with ease.  This will save you many near

heart attacks and spare the world a lot of harsh and ugly language.

PRINTING, PLOTTING, AND PRODUCTION

Producing your finished drawing is where things get crucial.  It's also where

AutoCAD shines, because A) it's a vector-based drawing package and will

reproduce your drawing at the highest resolution of your printer or plotter; B)

it supports a vast variety of output devices; and C) it can be made to output

your drawing to exactly the correct scale.

If you just want to get a rough-and-ready hard copy, you can use just about

anything to print out the drawing.  However, if your intent is to obtain

camera-ready output that can be used to create a photographic negative, such

as would be required to expose a negative-photo-resist copper circuit board,

you're almost certainly going to need either a plotter with very fine output

resolution (150 dots per inch or better) such as a laser printer; or you will

need to use a pen plotter.  We'll discuss both those options in a moment, but

first let's look at some other considerations.

Mirrors, Scales, and more...

As mentioned earlier, it's most common (and AutoCAD-PCB assumes) to draw printed

circuit boards from a top view, as if looking at the bottom traces through a

glass board.  But when it's time to print out the bottom traces and pads to make

a photographic negative, you need a mirror image of the drawing.  This is easy

to do with AutoCAD, although it can take a little time if it's a complex

drawing.  To keep from making any non-recoverable mistakes in this massive

alteration of the drawing, I generally exit to the DOS command line and make a

copy of my drawing file under a different name to do the mirroring with.  (EX:

C> COPY MYFILE.DWG MIRROR.DWG)  Once you have your drawing on the screen, enter

the command MIRROR <Ret>.  When it asks you to select the objects to mirror, you

can select your entire board by entering "W" (for Window) and then bracketing

the whole board with a window.  You can hit <ret> indicate that you're done

selecting objects to mirror.  AutoCAD then asks for the first and second points

of a mirror line; this is the line around which the drawing will be mirrored.

If you want to keep both the original drawing on screen and have the mirror

version next to it, pick a mirror line outside of your drawing of the board.  If

you want to mirror the drawing and have the original view deleted, you can pick

a mirror line that's right down the middle of the board.  AutoCAD then asks if

you want to delete the "old objects", by which it means all the stuff you're

trying to mirror.  Again, if you want to keep both the drawing and it's mirror

image, reply "N", but if you're mirroring the board "in place" by putting a

mirror line down the middle, you'd better answer "Y" or you'll have a very, very

big mess on the screen.  When you hit return, the program will churn away and

after a while (depending on the complexity of your drawing and the power of your

computer) you'll have your mirror image.  You may have succeeded in mirroring

the image right off the edge of the screen; if so, you can see everything you've

got by using the ZOOM EXTENTS command.

Now you have a view of the board from the bottom, perfect for making a negative

for the bottom.  However . . . you've probably still got some things like

component edges, and labels, and top-side traces or jumpers, none of which you

want to see on a drawing you're going to use to make your actual negative.  This

is where the use of layers gets very handy.  You can turn your TOP, LABELS, and

PARTS layers off with the menus or from the command line, and all you'll see are

the PADS and BOTTOM and OUTLINE, which is what you want.  Incidentally, before

you start printing or plotting, you'll also have to make sure you have FILL

turned ON and do a REGEN so that it will output nice solid lines, not just

outlines of your traces.

Obviously, if you're going to make a negative for the TOP of the board, you

don't have to mirror it.  You can just turn off the BOTTOM and PARTS and

LABELS layers and you'll be ready to go.  (Leave PADS on, though.  The TOP and

BOTTOM share the solder pads layer.)

Printers and Plotters

If you are trying to make a hard copy that will become a photographic negative

(or positive, if you're using positive-resist circuit boards), the whole idea is

to get output that is A) as FINE and SHARP a resolution as you can possibly get,

and   B) as UTTERLY, SOLIDLY BLACK as you can manage.  There are ways to enhance

both your resolution and your contrast, if this is a problem, as we'll see.  But

first let's look at some straightforward ways to get our artwork.  Please note

that again, you may want to have access to AutoCAD manuals to help you if you

need to change the configuration of AutoCAD to handle a different output device,

or to change the way it uses that output.  This is done with the "Configure

AutoCAD" command (5) available when you first start AutoCAD.

Laser Printers

Laser printers can give you both sharpness and blackness. With plenty of toner

and at 300 dots per inch, you can get camera ready, 1:1 artwork from a laser

printer.  However, please note that some Laser Printers, especially older HP

LaserJets, don't have enough memory to print an entire 8.5" by 11" page at

300 dots per inch.  In this case, you either have to keep your drawing size

to less than half a page (which is frequently possible), or you have to

run the printer at 150 dots per inch, which you can do by changing AutoCAD's

configuration.

I've been told that you can laser print-plot on vellum, which is translucent

to light, and use it as a positive for positive-resist boards directly,

without transferring it to film.  It's also possible to laser print

with special paper, plastic, and/or toner to make an image that is then

directly transferred to a non-sensitized copper board with a hot clothes

iron.  On someone's suggestion I have even tried this with plain paper and

ordinary toner, with mixed success, mostly on really small items.

I will definitely state that at UN-L's Engineering Electronics Shop, we've found

negative resist boards a whole lot more reliable to work with, as they are

less critical in their exposure and development than positive resist boards.

We HAVE attempted to laser print directly onto the kind of overhead transparency

sheets that are designed for use with photocopy machines.  YOU MUST HAVE THE

KIND OF SHEETS DESIGNED FOR PHOTOCOPIERS OR YOU CAN GET A BLACK, MELTED MASS

OF PLASTIC IN YOUR LASER PRINTER!!!  Anyway, it can work, but generally speaking

the resulting black-on-clear positive image is either not black enough, or may

contain a lot of "pinholes" where it's supposed to be black.  Little irreg-

ularities like that will drive you insane if you go ahead and make a board and

don't realize that pinholes have created traces with breaks in them.  On top of

all that, this does give you a positive image and like I said, we're not really

fond of positive resist etching processes.  BUT . . . it's a tempting way to go.

It might be worth at least a try.

[NOTE: I'll keep referring to "your negative", assuming that you are making a

drawing with black traces on white paper and then having a photo negative made

with clear traces on black.  If you are using a positive-etch board you will

need a POSITIVE film transparency made, with black traces on clear film.  Any

lab that can make one can make the other.  Be sure you get this straight with

whoever's making your "negative."]

Pen Plotters

The best laser printer can meet it's match from even a modest pen plotter.

Pen plotters can give you smooth lines without the slightest hint of the

"jaggies".  However, the user must remember the goals of "sharp and black"

and take some precautions when using pen plotters.  The most important one

is, obviously, to have a plotter pen that's very sharp and has plenty of

ink.  Reasonable results can be obtained with the common disposable felt-tip

plotter pens, if you use a nice new one.  However, for the real McCoy, try

to find a steel-tipped ink plotter pen that you can fill with India ink.

These are like fine-point drafting pens and give superior consistency of

line.  However, they're not always easy to find and they aren't cheap.  But

if you plan to do a lot of this kind of work, you won't regret getting one.

A nasty byproduct of plotters, however, is the possibility of the ink lines

spreading or smearing.  If you use ordinary paper, perhaps a sheet from your

computer's fan-fold printer paper, you're going to see your lines bleed all

over the place.  You need to find paper that's good and "hard".  There is

drafting paper available that has this quality; also, you'll find that a lot

of the inexpensive photocopy paper these days has one "hard" side for use

with copiers and laser printers.  You can actually feel the added smoothness

of the hardened side.

What If My Printer or Plotter Isn't So Great?

As we'll discuss in a moment, the general practice is to plot or printer-plot

your drawing out in the actual size, so a simple contact photo negative can be

made at a 1:1 ratio.  However, if your output resolution isn't so great - a

laser or dot matrix printer that's plenty black but only 150 dpi or so - you can

get around it by making your output exactly twice as large as what you'll need.

You then tell the people making your negative to shrink your image down by

EXACTLY half.  This is a common practice in professional manufacturing.

If you are having trouble getting the image black enough, try taking your

print or plot to a photocopy machine and duplicate it on that.  Maybe even

copy the copy, if need be.  This can sharpen and blacken your image, but it

can also lead to extra spots, holes, and other degenerations, so examine

your copies carefully.  You may need to touch them up by hand with a black

pen.

How about combining the last two suggestions at once - using a copy machine to

reduce your image in half while at the same time making it blacker?  Good idea,

EXCEPT . . . photocopier optics are not designed to give you PRECISE and EVEN

reduction or enlargement.  You'll probably end up with things that don't really

line up, which is disasterous when ICs and other parts require precise .1 inch

spacing.  You can give it a try, but make sure you set some of your parts onto

the output to see if it's actually the right size before you run off to have a

negative made.  This is a good idea regardless of how you print or plot your

drawing.

Here We Go ... Ready to Plot!

OK, having waded through all that, you're now ready to actually plot out your

drawing.  You can either choose Printer Plot a Drawing or Plot a Drawing from

the list of choices that appears when AutoCAD is first run; or you can enter

PRPLOT or PLOT from the command line if you are still editing a drawing.

AutoCAD will ask you just what you want to plot.  "Display" plots everything

that happens to be on the screen at the moment.  "Limits" plots to the limits of

the drawing, which we set to the default of 8.5 by 11 inches.  "Extents" only

plots what's inside of the farthest extent of the actual lines and artwork in

your drawing, i.e., everything.  "Window" lets you bracket a chunk of your

drawing and just plot it, which is useful if you're trying to plot just the

mirror image of the bottom you made, or you're trying to keep the area plotted

to less than half a page because you're using a memory-starved laser printer.

"View" refers to any particular views of the drawing you may have saved with the

VIEW command.

AutoCAD will then put up on the screen all of the print/plot options currently

in force and ask you if you want to change anything.  Generally speaking:

I've had very inconsistent results with plotting large drawings to a file

instead of directly to the printer: sometimes it works perfectly and is an

easy way to transport your output to a different machine that has a better

printer but doesn't have AutoCAD, but sometimes the resulting print has flaws

in it.

The size units should be inches.

The Plot origin can be 0,0.

The Plotting area should match whatever paper you're plotting on, usually

8.5 by 11.

With printer-plotting, you generally DO rotate 2-dimension plots 90 degrees;

with pen plotters, usually not.  It depends on your drawing.  Try it and see.

If you are printer-plotting on a LaserJet with 500k RAM you may want to NOT

rotate your drawing so it can all fit in the top half of a 8.5 x 11 inch paper

and thus be printed at 300 dots per inch.

Hidden lines should NOT be removed.  This is a 2 dimensional drawing that

shouldn't have any hidden lines.  If you answer yes, your computer could go

away for a very, very long time trying to hide some of your layers.

+==============================================================================+

|                            AND THE BIG ONE . . .                             |

| The scale of the drawing.  IF YOU WANT AN ACTUAL SIZE OUTPUT, THE SCALE OF   |

| PLOTTED INCHES = DRAWING UNITS MUST BE 1=1000.  If you want a double-size    |

| output so they can "Shoot it down" photographically by 1:2, you should give  |

| a scale of 2=1000 or 1=500.                                                  |

+==============================================================================+

However, if you're just plotting out a copy to look at, not etch, and you want

it nice and big so you can see all the details, you can answer "F" or "Fit" for

the scale, and AutoCAD will (theoretically) make the drawing fit your piece of

paper.  BUT . . . for some unfathomable reason, it seems that as often as not

when you tell AutoCAD to make the drawing "Fit", it will then come up and say

that the drawing will exceed the limits of the paper.  Real useful command,

right?  To this day I don't understand this.  The way to get around this is to

tell AutoCAD to plot an area larger than what you actually need plotted, for

example, tell it to plot everything inside a window, but then make sure you

include a wide margin around the thing you want to plot.  Then, if AutoCAD tries

to scale it to FIT and says that it'll be losing some of the drawing, it's just

going to lose some of that blank margin.

I've Got My Nice Sharp, Black Drawing - Now What?

Now you've got to mull over your options.  There are PC board labs that will

make your board for you if you just provide them with the artwork you've

created.  (And if you provide them with suitable quantities of cash.)  If you're

just producing a single board, you may find out that this is a pretty expensive

way to go, because there are set-up fees, and negatives to make, and possibly

minimum orders to fill, so the first board or boards end up costing quite a bit.

However, if you're making a whole lot of them, this is well worth looking into.

After the initial costs, most PC board outfits can then crank out large numbers

of your board at a very modest cost.  This is especially nice when they offer

such things as drilling out all of the holes, screening of labels or solder

resist, and plating-through the holes.  If you're going into limited production,

do check this out.  But many times, you just want one or two copies of your

board.  At this point, you should look into other options.  Check the ads in the

back of electronics magazines, check your local papers, ask around at Radio

Shacks, read the Yellow Pages, and see if there isn't a small business or local

hobbiest who will produce single boards or small quantities for a reasonable

cost.

If you're going to make your own PC board using a photo-negative-resist process,

you need to find someone to make you a nice, HIGH CONTRAST negative from your

artwork.  Our Shop uses the University of Nebraska-Lincoln's Photographic

Productions lab.  The kind of photo shop that just cranks hundreds of rolls of

color print film through a machine in an hour is probably not going to know what

you're talking about, but there are many other commercial places that will.  Try

custom photo printing places, professional camera stores, or even your local

professional photographer, who may do his own darkroom work.  One of these

places should be able to either help you or steer you to someone who can.

Once you've found this marvelous individual, tell them you need a HIGH CONTRAST

negative made.  If your drawing is actual size, make sure you specify that it

must be 1:1.  If you've made a double-sized drawing, make sure they know it must

be reduced by EXACTLY 2:1.  Emphasize that the black areas must be as solidly

black as possible.  These people aren't dumb: explain just what it is you're

doing and they'll understand the need for precision and high contrast.

And Then ? . . .

Once we get to exposing, developing, and etching the board, things get a little

less certain because there are different kinds of boards and different kinds of

chemicals.  Most photosensitized boards that you can buy locally will include

instructions regarding how long to expose them, and how to develop them.  Please

remember that sometimes these instructions are enclosed in the light-tight

package the circuit boards are sold in: DON'T OPEN THE PACKAGE IN THE LIGHT!

While PC boards are not as sensitive as photographic film, it doesn't take much

light to "nuke" your board and make it unusable.  Open the package in the dark

or in a safe-lighted room.  Sometimes you can buy the board and enough developer

to finish it all in a single inexpensive kit.  This is handy because you're sure

you've got the right stuff for the job and you don't have to invest in more

chemicals than you're going to use.

The procedure for exposing most boards is pretty much the same: you clamp your

negative or positive on top of the PC board in a dark room, then expose it to

ultraviolet light for a while.  The UV light in common flourescent bulbs will

expose many boards in 20 to 45 minutes.  It's better to get real UV lamps,

because you can cut your exposure time down to about two minutes, and because

fast, high-intensity exposure seems to be more forgiving of the quality of your

negative than slow, low-intensity exposure.

------------------------------------------------------------------------------

BE CAREFUL - SOME TYPES OF UV LIGHT ARE EXTREMELY HAZARDOUS TO THE CORNEAS

OF YOUR EYES!!!!!!!

------------------------------------------------------------------------------

Oh, and one major, major caution: MAKE SURE YOU PUT THE NEGATIVE ON THE RIGHT

WAY.  We've seen a lot of boards expose, develop and etch beautifully and then

discovered that the pattern was backwards because the negative was on upside

down.  How do you avoid this?  PUT TEXT ON YOUR TRACE LAYERS, remembering to

make it BACKWARDS on the bottom so it's forwards when you mirror the bottom.

When you expose the board, make sure you can read your text forwards.

The chemicals for developing and etching the boards are available locally

and also include instructions on how to use them.  Follow the instructions

religiously, as many of the chemicals involved in these processes are REALLY

NASTY to breath and/or touch.  Use good ventilation if you value your brain

cells and lungs.  And keep the etchant off of your hands or clothes - you won't

notice it at first but then your skin will start to burn.  Ouch.  Also, not only

will used etchant make indelible stains on your clothes but you'll find the

cloth starts to disintegrate when you wash it.  The most common etchant is some

nasty stuff called ferric chloride; if you can find it, try to get yourself some

sodium persulfate etchant.  Sodium comes as a powder you mix with water, is

transparent so you can see your board etch more clearly, and turns a nice bright

blue as it becomes saturated with copper so you know to throw it away.  (NOT

down your drain unless you want to eat holes in your plumbing.)

------------------------------------------------------------------------------

ONE MORE TIME: PRINTED CIRCUIT BOARD DEVELOPMENT AND ETCHING INVOLVES CHEMICALS

THAT CAN CAUSE SERIOUS INJURY IF INHALED, INGESTED, OR HANDLED IMPROPERLY. USE

* ALL * SAFETY PROCEDURES SPECIFIED BY THE MANUFACTURER AND KEEP ALL EQUIPMENT

AND MATERIALS SECURE FROM ACCESS BY CHILDREN OR OTHER NON-QUALIFIED PERSONS.

------------------------------------------------------------------------------

Our Little Advertisement . . .

Making the board sound a little daunting?  Just want to draw it and print it and

have some other poor fool play in the darkroom with the toxic stuff?  If you

are a UN-L student, staff, or faculty member, you should know that the UN-L

Engineering Electronics Shop has the facilities to etch PC boards if provided

with a negative and the photosensitized board.  The EE Shop tries to keep a few

small boards in stock to sell if you haven't got one you've purchased elsewhere.

We will expose, develop, and etch, and optionally tin-plate your board, for a

modest fee.  We have the necessary chemicals for developing negative-resist

boards; if you want to use a positive-resist board, you'll need to also provide

us with the chemicals for that board.  It's not uncommon for such chemicals to

be packaged in small kits,or bundled with a board, for the production of just

one PC board.

The EE Shop will NOT drill out the holes on your finished board.  It's hard to

overstate the amount of time required to drill boards - go ahead, count the

number of holes on your design!  Even a small board, such as a microcontroller

circuit, can end up with 400 to 1000 holes!  To drill the holes, it's best to

have the use of a drill press.  Even a drill press that just has a hand drill in

a movable clamp will work.  You'll also need a number of small drill bits,

ranging in size from #60 to #70 or so.  YOU NEED MORE THAN ONE, because YOU WILL

end up breaking one at some point.  The EE Shop WILL permit you to use our drill

presses, BUT you have to provide your own bits.  Also, as mentioned earlier, we

don't have the means to "plate-through" the holes on a double sided board.

THIS OFFER TO SERVE AS A BOARD ETCHER DOES NOT EXTENT OUTSIDE THE CONFINES OF

THE UNIVERSITY OF NEBRASKA - LINCOLN.

Closing comments:

In searching for a CAD tool for electronic design that was available to our

undergraduates, we've recently been evaluating the PADS software shareware

edition and found it has considerable advantages over using AutoCAD.  Still,

our tools may fill a niche where people already have AutoCAD installed.

If you are knowledgable about AutoCAD, you'll see that creating these tools

was not enormously difficult, although it WAS very time - consuming. Every time

I use them, it seems I find something I could have done better, or I think of

another predefined block I could have added; but I had to quit at some

point.  As I mentioned, I don't intend to add to these tools, leaving that

for others to do.  If you want to drop me a line, I'm accessible on the Int-

ernet at  mpaul@unlinfo.unl.edu or mpaul@engrs.unl.edu.  HOWEVER: I WILL NOT

serve as an on-line substitute for an AutoCAD manual, and I'm not interested

in tutoring anyone in the basic principals of AutoCAD when that information

is quite available elsewhere. If you don't have a manual or access to one,

you're probably not using a legal copy of AutoCAD!

Our shop is also a good source of general advice regarding various methods of

prototyping, circuit design, and other neighborly advice.  Stop by and see us

some time.

Paul Marxhausen

Electronics Tech III

Engineering Electronics Shop

Mail:     209N Walter Scott Engineering Center

Location: 122 Walter Scott Engineering Center

University of Nebraska-Lincoln

Lincoln, NE 68588-0511

Internet Email:   mpaul@unlinfo.unl.edu, mpaul@engrs.unl.edu