==================================================================

EEEEEEE  MM   MM  IIIIIII  L       Y     Y        5555555   22222

E        M M M M     I     L        Y   Y         5        2     2

E        M  M  M     I     L         Y Y          555555         2

EEEE     M     M     I     L          Y     ---         5   22222

E        M     M     I     L          Y                 5  2

E        M     M     I     L          Y           5     5  2

EEEEEEE  M     M  IIIIIII  LLLLLLL    Y            55555   2222222

==================================================================

A

Simulator/Emulator

for the

8051 and 8052

Release 1.3

Revised 04-Jan-94

Copyright 1991-1994 Dave Dunfield

All rights reserved

EMILY: An 8051/52 simulator                                      Page: 1

1. INTRODUCTION

EMILY is a software simulator for the 8051 and  8052  families  of

processors. It provides a powerful  debugging  environment,  offering

many features:

- Supports full 64K of PROGRAM and  64K  of  DATA  memory.  DATA  and

PROGRAM memory may also be overlapped into  a  single  64K  address

space.

- Hardware emulation mode communciates with the  target  system,  and

allows you to include the physical I/O lines, timers etc.  in  your

simulation.

- Single step, Multi Step, Animate and Full Speed execution modes.

- Simultanious on-screen displays of  program  disassembly,  internal

memory, CPU registers and simulation messages.

- Full screen editors for CPU registers, Special  Function  Registers

(SFR's), and each of the INTERNAL, EXTERNAL DATA and PROGRAM memory

spaces. All of the above may be viewed/altered at any  time  during

the debugging session.

- Multiple breakpoints are transparent to the user program.

- Supports the additional SFR's and internal RAM of the 8052 series.

1.1 System Requirements

EMILY will run on any IBM PC/AT/PS2 compatible  system  with  a

MDA/CGA/EGA or VGA display, two COMM ports, and at least  192K  of

free system RAM.

- If hardware emulation or serial I/O are not used, only one  COMM

port is required.

- If Overlapped data/program memory is used, only 128K of free ram

is required.

- Some CGA displays may exibit "snow" or flicker during high speed

screen updates. This effect will not occur with MDA, EGA or  VGA

displays.

EMILY: An 8051/52 simulator                                      Page: 2

2. STARTING UP EMILY

If you are using the optional hardware emulation mode, insure that

the resident control software (supplied) is inserted into the  target

system, and that the PC running EMILY  is  connected  to  the  target

systems serial port  via  COM2.  Power  on  the  test  system  before

starting EMILY. The resident control software  must  be  active  when

EMILY "looks" for it.

EMILY is invoked by typing her name at the DOS command  line.  You

will be greeted with a "welcome" screen which displays a reminder  of

the command line syntax and the copyright message. This  screen  will

remain on the display for about 5 seconds (10  seconds  in  the  demo

version). Pressing any key while this screen is visible  will  bypass

this delay, and proceed directly to the main control panel.

If EMILY is given a file name on the command line, she loads it in

the same manner as if the 'L'oad command had been  used  from  within

the main control panel.

EMILY: An 8051/52 simulator                                      Page: 3

EMILY also accepts several option switches on the command line:

/Hardware[:n]  - Enables the hardware emulation feature to cause

EMILY to interact with the target  system  when

performing I/O accesses.  This  option  may  be

followed by a  ':'  and  a  numeric value which

defines the BAUD RATE to be used to communicate

with the target system. Default is 28800.

/Mono *        - Inhibits the  color display, even  if  a  color

adapter is present. This option may  be  useful

if your monitor/adapter combination provides an

unpleasent or unusable display for EMILY.

/Overlap       - Joins EXTERNAL DATA  and PROGRAM  memory into a

single 64K address space which can be  accessed

with either MOVX or MOVC. This is equivalent to

a hardware implementation in which the PSEN and

RD signals are ANDed together.

/Reset         - Causes EMILY to de-assert the RTS line when she

initializes the serial port. This option may be

required when using  EMILY  with older versions

of our "BD52" single board computer.

/Swapcom       - Exchanges the operation of the two serial ports

on the PC.  When this option is used, COM1 will

be used to communicate with the  target system,

and COM2 is used for serial I/O redirection.

* Note: If some of the EMILY windows appear  to  be  an  un-readable

washed-out yellow color, try adjusting the color contrast on

your monitor. Those windows should be an easy to read brown.

If you are using the '/Hardware' option, and EMILY  is  unable  to

establish communication with the target system, you will see an error

message flashing on the screen. EMILY will keep trying, allowing  you

to debug the connection. You may also press the ESCAPE key to disable

hardware emulation, and proceed with a software only simulation.

EMILY: An 8051/52 simulator                                      Page: 4

3. USING EMILY

3.1 The Main Control Panel

Once EMILY is up and running, you will be  presented  with  the

"Main Control Panel", which consists of four "windows" displaying:

- A disassembly of program memory       (Upper LEFT)

- The contents of CPU registers         (Upper CENTER)

- The contents of INTERNAL MEMORY       (Upper RIGHT)

- Informational messages from EMILY     (Lower portion)

All of EMILY'S functions are  accessed  by  command  characters

entered from this screen:

3.1.1 A - Animate execution

This function begins execution of the user program (from the

address in the PC register). As each instruction  is  executed,

EMILY will display the  instruction  being  executed,  the  CPU

registers, and the contents of internal RAM in the  appropriate

windows.

When you enter the 'A' command, EMILY will  prompt  you  for

the number of instructions to animate. Pressing return (without

entering  a  number)  will  cause  the  animation  to   proceed

non-stop. Pressing <escape> will return you to the Main Control

Panel.

The animation will terminate when:

- The requested number of instructions have been stepped.

- A breakpoint is encountered.

- The illegal opcode ($A5) is encountered.

- The <escape> key is pressed on the PC console.

3.1.2 B - set Breakpoint

This command sets a breakpoint such that  program  execution

under the 'A'nimate, or  'G'o,  commands  will  stop  when  the

breakpoint address is encountered.

Up to 10 breakpoints may be in effect at the same time.

You will be prompted for  the  address  (in  HEX).  Pressing

return (without entering an address) will cause the  breakpoint

to be  set  at  the  current  program  counter  (PC)  location.

Pressing <escape> will return you to the main control panel.

EMILY stores the breakpoint addresses internally in a simple

zero terminated list. This allows  her  to  avoid  searching  a

fixed size table, and helps  speed  up  the  operation  of  the

program, especially when there are few breakpoints.  This  does

however cause the restriction that you cannot set a  breakpoint

at address zero ($0000).

EMILY: An 8051/52 simulator                                      Page: 5

3.1.3 C - Change register

You will be prompted for the CPU register to change. Enter a

character from the following list, ESCAPE to abort, or F1 for a

reminder:

A   - A accumulator

B   - B accumulator

D   - Data pointer (DPTR)

S   - Stack pointer

P   - Program counter

W   - processor status Word

0-7 - general register 0-7

After you have selected the register, you will  be  prompted

for the new contents (in HEX).

3.1.4 D - edit Data memory

This command allows you to edit EXTERNAL DATA  memory  using

EMILY's  built  in  memory  editor.  See  the  "Memory  Editor"

section.

3.1.5 F - display/edit special Function registers

The names and contents of all SFR's are displayed in a menu.

To change a register, enter the character preceeding  the  ':',

immediately to the left of its name. You will be  prompted  for

its new contents.

If multiple columns of SFR's are  defined,  use  <space>  to

select the column to edit.

When debugging  with  the  hardware  emulation  option,  the

values read/and written to the SFR's will  reflect  the  actual

state of the hardware lines etc. on the  target  system  board.

Pressing any key while in this menu will refresh  the  display,

and make visible any changes which have occured on  the  target

system I/O lines etc.

When debugging without the hardware emulator  option,  SFR's

are simply stored internaly within EMILY's memory.

EMILY: An 8051/52 simulator                                      Page: 6

3.1.6 G - Go: execute program

This  function  begins  high-speed  execution  of  the  user

program (from the address in the PC register).

When you enter the 'G' command, EMILY will  prompt  you  for

the number of instructions to execute. Pressing return (without

entering a number) will cause the program to proceed  non-stop.

Pressing <escape> will return you to the Main Control Panel.

Program execution will terminate when:

- The requested number of instructions have been executed.

- A breakpoint is encountered.

- The illegal opcode ($A5) is encountered.

- The <escape> key is pressed on the PC console. *

* NOTE: If you are using 'Local' serial I/O mode, ESCAPE offers

you a menu of execution I/O functions. See the F3  command  for

more information.

3.1.7 I - edit Internal memory

This command  allows  you  to  edit  INTERNAL  memory  using

EMILY's  built  in  memory  editor.  See  the  "Memory  Editor"

section.

3.1.8 J - Jump to PC

This command re-positions the disassembly display window (if

necessary) to display the instruction addressed by the  current

CPU Program Counter (PC).

3.1.9 K - Kill all breakpoints

Removes ALL breakpoints after prompting for permission to do

so.

3.1.10 L - Load program image

Prompts for the name of a file, and loads  it  into  PROGRAM

memory. The file must  be  in  either  INTEL  or  MOTOROLA  hex

download format.

3.1.11 N - Next disassembly window

Advances the disassembler to display the  following  screen.

Subsequent 'N' commands  will  proceed  to  disassemble  higher

memory addresses.

3.1.12 P - edit Program memory

This command allows you to edit PROGRAM memory using EMILY's

built in memory editor. See the "Memory Editor" section.

EMILY: An 8051/52 simulator                                      Page: 7

3.1.13 R - Remove a breakpoint

Prompts for the address, and if a breakpoint is set at  that

address, it is removed. Pressing return  (without  entering  an

address) will remove the  breakpoint  at  the  current  program

counter (PC) location.

3.1.14 T - Traceback viewer

In all execution modes, EMILY keeps track of the  last  4095

instructions executed. By entering the  traceback  viewer,  you

can examine this recording, and determine program  activity  up

to 4095 instructions preceeding the one on which execution last

stopped.

The following command keys are available in the viewer:

Up   - Backup the display 1 instruction.

Down - Advance the display 1 instruction.

PgUp - Backup the display 50 instructions.

PgDn - Advance the display 50 instructions.

Home - Position to the beginning of recording.

End  - Position to the end of recording.

F1   - Display a help summary.

F2   - Prompt for position & move.

F3   - Search backward (in time) for address. *

F4   - Search forward  (in time) for address. *

Esc  - Exit the Traceback viewer.

* If you press <return> without entering an address, the search

commands will look for the previous or next instance of the

instruction at your current position in the traceback buffer.

Only the instruction executed  is  saved  in  the  traceback

viewer, the contents of registers, RAM etc. are not recorded.

The instruction at position 0 in the traceback buffer is the

one to which the current  PC  points,  and  has  not  yet  been

executed.

3.1.15 V - Vector to interrupt

This function prompts you with a menu of the available  8052

interrupts. Use the arrow keys to position the  cursor  bar  to

the interrupt you desire to simulate, and press ENTER to select

it. You may use ESCAPE to cancel the function and return to the

Main Control Panel.

EMILY will simulate a long call (LCALL) to the corresponding

interrupt vector address by saving the current Program  Counter

on the CPU stack, and reloading it with  the  vector.  Any  SFR

bits normally set when the particular interrupt occurs will  be

set at this time.

EMILY: An 8051/52 simulator                                      Page: 8

3.1.16 F1 - Help display

Pressing this key displays a summary of the  EMILY  commands

which are available from the Main Control Panel.

3.1.17 F2 - Set Disassembler Address

Prompts for  an  address  (in  HEX),  and  re-positions  the

disassembler window to display instructions beginning  at  that

address.

3.1.18 F3 - Setup Serial Port

Opens a window displaying the (B)audrate,  (P)arity,  (D)ata

bits and (S)top bits which are  currently  configured  for  the

PC's COM1 serial port.

Press B, P, D or S to cycle to the next corresponding entry.

When Baudrate is selected as 'Local', EMILY  will  re-direct

the  programs  serial  accesses  to  the  message  window.  Any

characters which are written to SBUF  will  be  output  in  the

message window, and  any  character  which  are  typed  at  the

keyboard will appear as incomming serial data. When  this  mode

is active, Pressing ESCAPE in the 'G'o command brings up a menu

of execution options:

End simulation  - Stops the 'G'o command.

Send ESCAPE     - Send ESCAPE through as a charcter.

Zoom window     - Switch to full screen I/O window.

If 'Local' is not selected, EMILY  re-directs  the  programs

serial accesses to the PC's  COM1  serial  port,  however,  for

maximum  flexibility,  the  actual  data  format  used  may  be

configured by this menu.

EMILY allows you to "fine tune" a baud rate,  by  using  the

Up/Down keys. Note that the  available  baud  rate  values  are

determined by the PC COMM  port  hardware,  and  may  not  fall

exactly on the value you wish to use.

Asynchronous communication is specified to work properly  as

long as the device is operating within 5% of the intended  baud

rate.

For example:

Desired baud rate       : 1500

Allowable range (+/- 5%): 1425-1575

Closest PC baud rate    : 1496

In the above example, the baud rate produced is within  0.3%

of  the  desired  value,   and   is   therefore   well   within

specifications.

EMILY: An 8051/52 simulator                                      Page: 9

3.1.19 <space> - Single step one instruction

Every time you press the <space> bar, EMILY will execute one

instruction, and update the on-screen displays.

3.1.20 PgDn - Advance IRAM display page

Pressing this  key  advances  the  internal  memory  display

window by one page (64 bytes).

3.1.21 PgUp - Backup IRAM display page

Pressing this key  backs  up  the  internal  memory  display

window by one page (64 bytes).

3.1.22 Down - Advance IRAM display line

Pressing the "down arrow" key advances the  internal  memory

display window by one line (4 bytes).

3.1.23 Up - Backup IRAM display line

Pressing the "up arrow" key backs  up  the  internal  memory

display window by one line (4 bytes).

EMILY: An 8051/52 simulator                                      Page: 10

3.2 The Memory Editor

When you activate the 'D', 'I' or 'P' commands  from  the  main

control panel, EMILY will invoke its internal "Memory  Editor"  to

access the desired memory area. When you are within  this  editor,

the following commands are available:

3.2.1 F1 - Help display

Pressing this key displays a summary of the  EMILY  commands

which are available from within the Memory Editor.

3.2.2 F2 - Set Edit Address

Prompts for an address (in HEX), and re-positions the editor

display and cursor to edit at that address.

3.2.3 F3 - Toggle Edit Mode

EMILY allows two edit modes, HEX  and  ASCII.  When  in  HEX

mode, your cursor will be positioned in the hexidecimal display

portion of the memory  memory  dump,  allowing  you  to  change

memory by entering a new value as two  hexidecimal  characters.

When in ASCII mode, your cursor will  be  positioned  over  the

ASCII portion of the dump, and  you  can  replace  memory  with

ASCII text typed at the keyboard.

Pressing F3 will toggle between HEX and ASCII mode,  causing

your cursor to switch between the corresponding portion of  the

memory display.

3.2.4 Up - Move up 1 line

Pressing the UP ARROW causes the cursor to move  upwards  on

the screen by one line. This reduces the memory edit address by

16 bytes.

3.2.5 Down - Move down 1 line

Pressing the DOWN ARROW causes the cursor to move  downwards

on the screen by one  line.  This  increases  the  memory  edit

address by 16 bytes.

3.2.6 Left - Move left 1 byte

Pressing the LEFT ARROW causes the cursor  to  move  to  the

left by one byte position. This reduces the memory edit address

by 1 byte.

If the cursor is already at the left hand  margin,  it  will

wrap around to the rightmost position of the previous line.

EMILY: An 8051/52 simulator                                      Page: 11

3.2.7 Right - Move right 1 byte

Pressing the RIGHT ARROW causes the cursor to  move  to  the

right by one byte position. This in increases the  memory  edit

address by 1 byte.

If the cursor is already at the right hand margin,  it  will

wrap around to the leftmost position of the following line.

3.2.8 PgUp - Move back 1 page

Pressing the PGUP key causes the editor  to  move  back  one

display page. This reduces  the  memory  edit  address  by  256

bytes.

3.2.9 PgDn - Move forward 1 page

Pressing the PGDN causes the  editor  to  move  forward  one

display page. This increases the memory  edit  address  by  256

bytes.

3.2.10 Home - Move to start of line

Pressing the HOME key causes  the  cursor  to  move  to  the

leftmost position of the line it is currently on. If the cursor

was already in the  leftmost  position,  it  is  moved  to  the

leftmost position of the preceeding line.

3.2.11 End - Move to end of line

Pressing the END key  causes  the  cursor  to  move  to  the

rightmost position of the line  it  is  currently  on.  If  the

cursor was already in the rightmost position, it  is  moved  to

the rightmost position of the following line.

3.2.12 Ctrl-PgUp - Move to start of memory

Pressing the CTRL and  PGUP  keys  re-positions  the  editor

display and cursor to address $0000 at the start of the  memory

block.

3.2.13 Ctrl-PgDn - Move to end of memory

Pressing the CTRL and  PGDN  keys  re=positions  the  editor

display and cursor to the  highest  address  available  in  the

memory block.

EMILY: An 8051/52 simulator                                      Page: 12

3.3 Hardware Emulation

EMILY supports a hardware emulation mode, which provides access

to most of the physical I/O ports, timers  etc.  from  the  target

system in your simulation.

The  target  system  must  be  running  the  supplied   control

software, which interacts with EMILY, and perform the I/O accesses

upon her commands. The serial port of the target  system  must  be

connected to the COM2 serial port of the PC.

There are a few restrictions which apply:

- The target system must provide access to the CPU serial port.

- There is an approximatly 0.7 MS delay when accessing the  I/O

ports and other physical hardware. This  occurs  due  to  the

serial activity required to communicate the data to and  from

the target system.  For  most  applications,  this  delay  is

acceptable, particularly in  the  stop/start  environment  of

simulation and debugging. NOTE: If you are using  a  baudrate

to communicate with the target system which is lower than the

default (28800), this delay will be longer.

- The control software uses TIMER1 internally to  generate  the

baud rate clock for its serial port. This means  that  TIMER1

is unavailable during the  simulation.  If  your  application

uses the serial port, we recommend you also use TIMER1 as the

baud rate generator. Since EMILY has a  separate  setup  memu

for serial port speed, parity etc. It will  not  affect  your

program that TIMER1 is disabled during the simulation. Simply

press F3 at EMILY's main control panel, and select the serial

parameters you desire.

EMILY: An 8051/52 simulator                                      Page: 13

When using this  (Hardware  Emulation)  feature,  the  Resident

Control Firmware should be running in  the  target  system  BEFORE

executing EMILY. Be sure to specify '/H:<speed>' parameter on  the

command line when starting EMILY.

The value of <speed> depends on the  target  systems  operating

frequency. The following table shows the speeds which can be  used

with several common  crystal  frequencies  without  modifying  the

supplied Resident Control Firmware:

Freq (Mhz)          Speed -> Actual

----------------------------------------

11.0592             28800 -> 28800  0.0%

11.0                28646 -> 28800  0.5%

9.0                 23438 -> 23040  1.7%

7.3728              19200 -> 19200  0.0%

5.0                 13020 -> 12800  1.7%

4.0                 10417 -> 10473  0.5%

3.68                9600  -> 9600   0.0%

3.59                9349  -> 9600   2.7%

3.0                 7813  -> 7680   1.7%

2.0                 5208  -> 5236   0.5%

1.84                4800  -> 4800   0.0%

1.0                 2604  -> 2618   0.5%

At the beginning of the supplied KERNAL.ASM  file  is  a  large

comment which describes how to change the baud  rate  constant  to

obtain other serial port speeds. The following table shows  speeds

which can  be  used  with  other  common  crystal  frequencies  by

altering the "BAUD" value in the KERNAL.ASM file:

Freq (Mhz)  BAUD Value      Speed -> PC Speed

------------------------------------------------

12.0            3           10417 -> 10473  0.5%

10.0            2           13020 -> 12800  1.7%

8.0             2           10417 -> 10473  0.5%

7.0             2           9116  -> 8862   2.8%

6.0             2           7813  -> 7680   1.7%

EMILY: An 8051/52 simulator                                      Page: 14

3.4 Messages

During the course of operation, EMILY may display the following

messages in the message window located in the lower portion of the

screen:

3.4.1 All Used!

You have tried to enter a  breakpoint,  when  all  available

breakpoints are in use. EMILY allows up to 10 code  breakpoints

to be active at any one time.

3.4.2 Animating at XXXX

Indicates that EMILY has  commenced  animated  execution  of

your program, beginning at the address 'XXXX'.

3.4.3 Already set!

Issued to indicate that the address at which you have  tried

to set a breakpoint is already active as a breakpoint.

This message will also  result  of  you  attempt  to  set  a

breakpoint at address zero ($0000).

3.4.4 Breakpoints: [XXXX ...]

This message shows the currently set  breakpoint  addresses,

and is issued whenever any  command  involving  breakpoints  is

performed.

3.4.5 Executing at XXXX

Indicates that EMILY has commenced full  speed  exection  of

your program, beginning at the address 'XXXX'.

3.4.6 Code and data are overlapped

This message results from the use of the  '/Overlap'  switch

on the EMILY command line, and indicates that the external CODE

and DATA memory address  spaces  are  combined  into  a  single

overlapping 64K segment.

3.4.7 Code and data are separate

Indicate that EMILY  has  established  separate  64K  memory

address spaces for the code and data, resulting in  128K  total

external memory.

3.4.8 FILENAME: Bad checksum

EMILY  has  encountered  a  record  containing  a  incorrect

checksum value while attempting to 'L'oad the named file.

EMILY: An 8051/52 simulator                                      Page: 15

3.4.9 FILENAME: Demo limit exceeded!

This message occurs only in the "Demonstration"  version  of

EMILY, and indicates that you have  attempted  to  'L'oad  more

than 500 bytes of code. The demonstration release  is  intended

only to allow you to  check  out  EMILY's  features,  you  must

purchase the "production" version if  you  intend  to  use  the

program for serious development and debugging.

3.4.10 FILENAME: Invalid HEX record

EMILY  has  encountered  an  improperly  formatted  download

record while attempting to 'L'oad the named file.

3.4.11 FILENAME: N bytes loaded

The  indicated  number  of  bytes  have  been   successfully

'L'oaded from the named file.

3.4.12 FILENAME: Unable to access

EMILY was unable to open and read the named file.

3.4.13 Hardware emulation is disabled

Indicates that the EMILY  is  running  as  a  software  only

simulator, and will emulate special  functions  registers  etc.

using internal RAM memory.

3.4.14 Hardware emulation is enabled

Issued if the "/Hardware"  switch  was  used  on  the  EMILY

command line, and the Resident Control Software in  the  target

system has been detected. In this mode  of  operation,  special

function register accesses  are  communicated  to  the  control

software, and result in physical activity on the target system.

3.4.15 Stopped at XXXX

Indicates that EMILY has stopped execution of you program at

the indicated address. This message is appended to the  initial

message indicating that execution had commenced.

3.4.16 Target system is not responding!!!

Indicates that the "/Hardware" option switch was used on the

EMILY command line, but EMILY  cannot  establish  communication

with the target system. EMILY will keep trying, allowing you to

attempt to debug the connection. You may also press the  ESCAPE

key, to disable hardware emulation mode,  and  proceed  with  a

software only simulation.

EMILY: An 8051/52 simulator                                      Page: 16

3.4.17 XXXX: Not found

This message is issued while in the 'T'raceback  viewer,  to

indicate that an address 'XXXX' you have searched for does  not

occur between your current position, and the start/end  of  the

traceback buffer.

EMILY: An 8051/52 simulator

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

1.1 System Requirements                                             1

2. STARTING UP EMILY                                                   2

3. USING EMILY                                                         4

3.1 The Main Control Panel                                          4

3.2 The Memory Editor                                              10

3.3 Hardware Emulation                                             12

3.4 Messages                                                       14