The NETSANE - Protocol :

1.) Preface :

=============

Netsane is intended to bring SANE (Scanner Access Now Easy) to the

network.

Why this ?

There are several imaginable situations, where you would like to access

an image-source across the network :

1) You need access to a scanner which has to be to be attached to a machine

(for physical or administrative reasons) you have no account on.

2) You want to provide public access to a picture source while allowing the

user to use the enhanced capabilities of SANE to control the device

generating the pictures. This would allow for SpyCams than can be easily

controlled online (i.e. you can have buttons/sliders for zooming and

looking around) without the overhead of conventional solutions (like

HTTP/CGI).

2.) The Protocol :

==================

2.1.) General Information

-------------------------

Netsane runs as a TCP service on the (prviledged ?) Port ????.

For now we have chosen to run on 6566/tcp. Please notify us, if we

conflict with something important.

I doesn't seem to be necessary anymore to run on a priviledged port,

as NETSANE can handle two-way cryptographic authorization.

2.2.) Helper Library and Parameter Transfer

===========================================

SANE makes use of lots of parameters. They are transferred in the

following way :

- Structures are broken up into their individual members.

This is done recursively if necessary.

- Each complete command is sent as one block of the form length,value.

- Opaque Values (like SANE_HANDLE) are transferred "as_is" using the

"%*c" format. E.g. sanei_printf("%*c",sizeof(handle),&handle); .

- INT Values in SANE are are transferred as " " (=SPACE) separated

strings in which hold the decimal representation in ASCII encoding.

I.e. bla=1234;sanei_printf("%d",&bla); results in "1234 " being added

to the output-string.

- Strings are transferred as a follows :

First a decimal encoded int (as above) which is the length of the string

including the \0 termination character. A NULL string is encoded

as length=0.

After a separator ("|") the string follows, but with the '\0' removed.

A separating " " (=SPACE) is added after that.

I.e. sanei_printf("%s","Hello world!"); results in "13|Hello world! ".

The used character set is assumed to be latin1.

It is discouraged to use characters other than ASCII to be

compatible. As "long-range" network connections are not

the primary concern of SANE, you may choose to send strings

in your local encoding, though this should only be done, if

ASCII or latin1 are not sufficient.

- enum or #define values are encoded as INT and transferred

this way. Please note, that this makes it necessary, that

sane.h is identical in this respect on all systems and only

extended in a compatible way.

The example implementation contains routines that allow accessing the

SANE data transfer just like printf/scanf.

sanei_printf(int fd,char *param_string,int command,...);

sanei_scanf (int fd,char *param_string,int *command,...);

Allowed format descriptors for these functions are :

%d	: Send/Recv a decimal value. I.e. a SANE_Int

%c      : Send/Recv a single raw byte.

%s      : Send/Recv a \0 terminated string.

All (! printf too !) functions expect pointers to the corresponding data

as the argument.

Modifiers :

*	: argument2 points to an array. Send argument1 fields of it.

(e.g. int field[2];sanei_printf("%*d",2,field);

Invalid with %s.

&       : Only valid with %s in sanei_scanf. The corresponding argument

is of type char ** - malloc space for the string and return

the address. This is there to allow for receiving NULL

pointers and for the common case of having to malloc the space

anyway.

The corresponding option-string is included with every command.

Please note, that the command is treated as a normal INT Argument

and only "special-cased" here for clarity.

2.3.) CONNECTION SETUP (mandatory)

==================================

After it has established a connection, the server will issue a greeting

message. It is allowed to send several lines of greeting :

S: CMD_HELO [optional text]

Syntax: "%d %s\n",CMD_HELO,"Some greeting message"

The client should reply with a HELO message itself (e.g. for logging

puposes), but this may be skipped.

The server is now in "command-stage". However it will normally require

authentication. This can happen anytime during a NETSANE session, so your

client should probably work event-oriented. (The protocol is designed to

deliver all information necessary to process a given request within the

request itself.)

The same applies for cryto-negotiation which can be requested any time.

2.4.) AUTHENTICATION (optional)

===============================

If authentication is enabled, the client is now asked to log on

by the server :

S: CMD_USER

This is done by first sending a username with the USER command.

C: USER myusername

Sytax: "%d %s\n",CMD_USER,"My_Username"

The server may now ask for a pass-phrase which can be computed in

the following ways :

2.4.1.) "classic" Unix : (mandatory)

------------------------------------

The password is transferred in plaintext and compared to a crypted

version of it on the server.

S: CMD_PASS_REQ PASS_UNIX

Syntax: "%d %d\n",CMD_PASS_REQ,PASS_UNIX

C: CMD_PASS_RPLY PASS_UNIX password

Syntax: "%d %d %s\n",CMD_PASS_RPLY,PASS_UNIX,"My password"

This authentication method is only recommended for "safe" networks,

where you can safely assume, that no spies can catch network packages.

2.4.2.) "crypt" : (mandatory)

-----------------------------

The password must be crypted by the client with three seeds which

are provided by the server. The system works as follows :

The server has a crypt()ed version of the password stored.

The client uses crypt() to achieve the same data by using crypt()

with the same seed (#1 sent by the server). ( This stage is to be

able to send an already crypted password to the site-admin, so even

(s)he doesn' know the plaintext).

Both systems then use crypt(crypted_passwd+2,seed#2) and

crypt(crypted_passwd+5,seed#3) to generate two passwords

which are then compared over the net. An example :

S: CMD_PASS_REQ PASS_CRYPT AB CD EF

Syntax: "%d %d %s %s %s\n",CMD_PASS_REQ,PASS_CRYPT,"Seed1","Seed2","Seed3"

The client now uses the password it is given (password) to calculate

the password stored on the server : ABRCL9ijBr2LY

Both systems then calculate crypt("RCL9ijBr2LY","CD")=CDcC3uINqJo7Q

and crypt("9ijBr2LY","EF")=EFIBfTmJjGhtM .

C: CMD_PASS_RPLY PASS_CRYPT CDcC3uINqJo7Q EFIBfTmJjGhtM

Syntax: "%d %d %s %s\n",CMD_PASS_RPLY,PASS_CRYPT,"Response1","Response2"

This scheme should reduce the risk of network-spies, but it will

not completely eliminate it. Note that only 24 bits are used for

distinguishing different accesses. I.e. a network spy has a

2^24 : number of monitored transmissions  chance to have the

right password.

Hopefully this scheme does not introduce a vulnerability for the

(crypted) password itself, as parts of it are encrypted twice.

I do not think the crypt algorithm is weakened very much by the

knowledge, that the last 5 letters of crypt A are the same as

the first 5 of crypt B.

2.4.3. More protocols

---------------------

can be implemented. They are identified by the second field of

the PASS request. (Any suggestions ?)

2.4.4. Authentication failure

-----------------------------

If authentication fails, the server will go back to stage 2) and

re-ask for USER and PASS. After a number of unsuccessful tries,

the connection should be terminated.

The client may ask the server for another authentication method

by replying

C: CMD_FAIL CMD_PASS [Allowed protocols]

Syntax: "%d %d %*d\n",CMD_FAIL,CMD_PASS,NumOfProtocols,&Protlist,...

In this case the server may choose to send the PASS again with

another protocol (should be configureable on the server side)

or to count the attempt as failed and go to Stage 2).

2.5. Crypto negotiation (optional)

==================================

No crypto-methods are used up to now. In case it is supported,

the protocol is as follows :

S: CRPT_REQ [allowed protocols]

Syntax: "%d %*d\n",CMD_CRPT_REQ,NumOfProtocols,&Protlist,...

The client MUST choose one of the allowed protocols (or NONE, if

this "protocol" is in the list) by:

C: CRPT chosen_protocol

Syntax: "%d %d\n",CMD_CRPT_RPLY,Protokol

All further communication will use the chose protocol.

Each protocol may exchange the necessary setup-data using a

protocol-specific extension.

2.6.) Command Stage

===================

The Sane server will go to command stage after authentication.

No more authority-checking is done. I.e. the connection could be

captured using IP-spoofing, but this is probably not that

critical.

No crypto-methods are used

The following commands are defined which closely map to the SANE

calls :

INIT	sane_init

EXIT	sane_exit

GDEV	sane_get_devices

OPEN	sane_open

CLOS	sane_close

GOPT	sane_get_option_descriptor

COPT	sane_control_option

PARM	sane_get_parameters

STAR	sane_start

READ	sane_read

ABRT	sane_cancel

IOMO	sane_set_io_mode

GSFD	sane_get_select_fd

STRS	sane_strstatus

2.7. The individual commands

============================

2.7.1. sane_init

----------------

INIT	sane_init

This should be the first command to call after authentication.

It will return the version of SANE in use on the server side.

Syntax: "%d\n",CMD_INIT

Reply : "%d %d %d\n",CMD_INIT,sane_rc,sane_version_code

2.7.2. sane_exit

----------------

EXIT	sane_exit

This should be the last command in command stage

Syntax: "%d\n",CMD_EXIT

Reply : "%d\n",CMD_EXIT

2.7.3. sane_strstatus

---------------------

STRS	sane_strstatus

Convert a sane_status reply to a string.

Using this function is optional. The server is required to

implement it, but the client may choose to resolve error numbers

to strings locally.

Syntax: "%d %d\n",CMD_STRS,sane_rc

Reply : "%d %s\n",CMD_STRS,"Descriptive error message"

2.7.4. sane_get_devices

-----------------------

GDEV	sane_get_devices

Get the supported devices.

Syntax: "%d\n",CMD_GDEV

Reply : "%d %d %s %s %s %s ...\n",CMD_GDEV,

sane_rc,dev1.name,dev1.model,dev1...,dev1.,  ... , ...

Please note, that ns_scanf returns 0, if no more data can be read from the

current message, thus you can detect when the list is complete.

2.7.5. sane_open

----------------

OPEN	sane_open

Open a sane_device.

Syntax: "%d %s\n",CMD_OPEN,devname

Reply : "%d %d %*c\n",CMD_OPEN,sane_rc,sizeof(SANE_Handle),SANE_Handle

The returned handle is not guaranteed to be identical with the one

the "real" backend has sent, the server may map them around.

2.7.6. sane_close

-----------------

CLOS	sane_close

Close a sane_device.

Syntax: "%d %*c\n",CMD_CLOS,sizeof(SANE_Handle),SANE_Handle

Reply : "%d\n",CMD_CLOS

2.7.7. sane_get_option_descriptor

---------------------------------

GOPT	sane_get_option_descriptor

Get an option descriptor

Syntax: "%d %*c %d\n",CMD_GOPT,sizeof(SANE_Handle),SANE_Handle,optnum

Reply : "%d %s %s %s  %d %d %d %d  %d",CMD_GOPT,

name,title,desc, type,unit,size,cap, constraint_type

The constraint-field is then transferred (in the same message) as a

series of "%s","%d" or "%d %d %d" type strings depending on constraint-

type. Transmission is terminated by "\n".

2.7.8. sane_control_option

--------------------------

COPT	sane_control_option

Get/Set an option.

Syntax: "%d %*c %d %d %d",CMD_COPT,sizeof(SANE_Handle),SANE_Handle,

optnum,action,type

If (action==SANE_ACTION_SET_VALUE) we now transfer the value:

"%d" for BOOL,INT,FIXED

"%s" for STRING,GROUP

"" (i.e. nothing) for BUTTON

Reply : "%d %d %d %d",CMD_COPT,rc,info,valueType

and then the value, as above

2.7.9. sane_get_parameters

--------------------------

PARM	sane_get_parameters

Get the scan parameters.

Syntax: "%d %*c\n",CMD_PARM,sizeof(SANE_Handle),SANE_Handle

Reply : "%d %d %d %d %d %d %d %d\n",CMD_PARM,rc,

format,first_frame,bytes_per_line,pixels_per_line,

lines,depth

2.7.10. sane_start

------------------

STAR	sane_start

Start scanning.

Syntax: "%d %*c\n",CMD_STAR,sizeof(SANE_Handle),SANE_Handle

Reply : "%d %d\n" ,CMD_STAR,rc

2.7.11. sane_read

-----------------

READ	sane_read

Get the scanner data.

Syntax: "%d %*c %d\n",CMD_READ,sizeof(SANE_Handle),SANE_Handle,maxlen

Reply : "%d %d %d %*c\n",CMD_READ,rc,length,length,data

2.7.12. sane_cancel

-------------------

ABRT	sane_cancel

Abort running scan.

Syntax: "%d %*c\n",CMD_ABRT,sizeof(SANE_Handle),SANE_Handle

Reply : NONE !! (This is an asynchronous event. ACK'ing it would complicate

things.)

2.7.13. sane_set_io_mode

------------------------

IOMO	sane_set_io_mode

Set the IO parameters.

Syntax: "%d %*c %d\n",CMD_IOMO,sizeof(SANE_Handle),SANE_Handle,nonblock

Reply : "%d %d\n",CMD_READ,rc

2.7.14. sane_get_select_fd

--------------------------

GSFD	sane_get_select_fd

Get a fd for select()ing ... this must be emulated. Transferring makes no

sense.

2.8. Logging out

The sane server will react to a connection-close by stopping all

current activities and cleanly shutting down the scanner interface.

This way of terminating a session is discouraged and included for

compatibility/network-failure.

The same applies for a timeout in the server which will disconnect

after a certain time of silence on the link (currently 5 Minutes).

This is done to keep hanging connections from blocking ressources

(especially the scanner ;-) indefinitely.

The normal way of logging out should be to shutdown SANE as if using

the library (call sane_cancel and sane_exit) and then issue the command

QUIT

This command will result in the same behaviour as a connection close

(which will be induced by the server, then), i.e. the server has to

shut down the scanner cleanly, no matter what state it is in.