February 10, 1999

To whom it may concern:

The following documentation is to address the issue suspend/resume in a DOS box.

For power management under Windows 95/98, ESS audio drivers comply with the APM 1.2, ACPI 1.0, and PPMI 1.0 specifications.  ESS audio drivers support standby/resume, as well as hibernation (suspend to disk)/resume.

For power management under Native DOS, ESS provides sample codes for customers to build into their BIOS to support such features.  However, ESS does not guarantee full functionality under such conditions (i.e. suspend during DOS game play).  To be able to support Native DOS suspend/resume requires the BIOS to fully restore the DMA controller as well as the PCI audio device.  Under certain circumstances, the coding that the BIOS will need to implement can get rather complicated.

For Windows 95/98 DOS box, a certain portion of the hardware must work together to maintain Sound Blaster compatibility.  While the Windows 95/98 DOS box is not a true Native DOS environment, similarities do exist between the two.  Therefore supporting power management under Windows 95/98 DOS box must carry the same limitations as that under native DOS.

The limitations under Native DOS/DOS box suspend/resume have two major factors.  One is the fact that DOS games are not power management aware.  DOS games have no mechanism for detecting suspend messages, and further more, no routines for stopping any FM or wav playback.  The other major factor is that any PCI audio device has no way to save and restore DMA context, be it from the device or the chipset.  The chipset DMA context lies outside of the PCI device domain responsibilities and therefore must rely on BIOS.  ESS PCI devices also do not support shadowing registers, and therefore has no means for supporting the saving and restoring of its own DMA context.  As a result, after a suspend/resume operation, it is likely that the user may experience a loss of FM sound, digital sound, or even the device configuration settings.

I hope that this answers any questions that you may have had with regards to power management capabilities of ESS PCI audio drivers in the Windows 95/98 DOS box environment.  If you have further questions, I can be reached at (510) 492-1719 or by e-mail at  howard.kim@esstech.com.

Sincerely,

Howard Kim

Technical Marketing Group, Audio

ESS Technology, Inc.