From: sunsite.unc.edu: /pub/Linux/X11/devel/supervga-info/vga.doc

(XFree86 distribution files)

-----------------------------------------------------------------

3C0h:  Attribute Controller: Address register

bit 0-4  Address of data register to write to port 3C0h

or read from port 3C1h (Reads only on VGA).

5  If set screen output is enabled and the palette can not be modified,

if clear screen output is disabled and the palette can be modified.

Port 3C0h is special in that it is both address and data-write

register. Data reads happen from port 3C1h. An internal

flip-flop remembers whether it is currently acting as

address or data register.

Accesses to the attribute controller must be seperated by

at least 250ns.

Reading port 3dAh will reset the flip-flop to address mode.

3C0h index 0-Fh  (r/W): Attribute: Palette

bit 0  (EGA) Primary Blue

1  (EGA) Primary Green

2  (EGA) Primary Red

3  (EGA) Secondary Blue

4  (EGA) Secondary Green

5  (EGA) Secondary Red

0-5  (VGA) Index into the 256 color DAC table.

May be modified by 3C0h index 10h and 14h.

3C0h index 10h (r/W): Attribute: Mode Control Register

bit 0  Graphics mode if set, Alphanumeric mode else.

1  Monochrome mode if set, color mode else.

2  9-bit wide caharcters if set.

The 9th bit of characters C0h-DFh will be the same as

the 8th bit. Otherwise it will be the background color.

3  If set Attribute bit 7 is blinking, else high intensity.

5  (VGA Only) If set the PEL panning register (3C0h index 13h)

is temporarily set to 0 from when the line

compare causes a wrap around until the next

vertical retrace when the register is automatically

reloaded with the old value, else the PEL

panning register ignores line compares.

6  (VGA Only) If set pixels are 8 bits wide.

Used in 256 color modes.

7  (VGA Only) If set bit 4-5 of the index into the DAC table

are taken from port 3C0h index 14h bit 0-1,

else the bits in the palette register are used.

3C0h index 11h (r/W): Attribute: Overscan Color Register.

bit 0-5  Color of screen border. Color is defined as in the

palette regsiters.

Note: The EGA requires the Overscan color to be 0 in high resolution

modes.

3C0h index 12h (r/W): Attribute: Color Plane Enable Register

bit 0  Bit plane 0 is enabled if set.

1  Bit plane 1 is enabled if set.

2  Bit plane 2 is enabled if set.

3  Bit plane 3 is enabled if set.

4-5  Video Status MUX. Diagnostics use only.

Two attribute bits appear on bits 4 and 5 of the Input

Status Register 1 (3dAh).

Value       EGA            VGA

0      Red/Blue        Bit 2/Bit 0

1      Blue'/Green     Bit 5/Bit 4

2      Red'/Green'     Bit 3/Bit 1

3                      Bit 7/Bit 6

3C0h index 13h (r/W): Attribute: Horizontal PEL Panning Register

bit 0-3  Indicates number of pixels to shift the display left

Value  9bit textmode   256color mode   Other modes

0          1               0              0

1          2              n/a             1

2          3               1              2

3          4              n/a             3

4          5               2              4

5          6              n/a             5

6          7               3              6

7          8              n/a             7

8          0              n/a            n/a

3C0h index 14h (r/W): Attribute: Color Select Register  (VGA Only)

bit 0-1  If 3C0h index 10h bit 7 is set these 2 bits are used

as bits 4-5 of the index into the DAC table.

2-3  These 2 bits are used as bit 6-7 of the index into the

DAC table.

Note: this register does not affect 256 color modes.

3C2h (R): Input Status #0 Register

bit 4  Status of the switch selected by the Miscellaneous Output

Register 3C2h bit 2-3. Switch high if set.

5  (EGA Only) Pin 19 of the Feature Connector (FEAT0)

is high if set

6  (EGA Only) Pin 17 of the Feature Connector (FEAT1)

is high if set

7  (EGA Only ??) If set IRQ 2 has happened due to Vertical

Retrace. Should be cleared by IRQ 2 interrupt routine

by clearing port 3d4h index 11h bit 4.

3C2h (W): Miscellaneous Output Register

bit 0  If set Color Emulation. Base Address=3Dxh

else Mono Emulation. Base Address=3Bxh.

1  Enable CPU Access to video memory if set

2-3  Clock Select

0: 14MHz(EGA)     25MHz(VGA)

1: 16MHz(EGA)     28MHz(VGA)

2: External(EGA)  Reserved(VGA)

4  (EGA Only) Disable internal video drivers if set

5  When in Odd/Even modes Select High 64k bank if set

6  Horizontal Sync Polarity. Negative if set

7  Vertical Sync Polarity. Negative if set

Bit 6-7 indicates the number of lines on the display:

0=200(EGA)  Reserved(VGA)

1=          400(VGA)

2=350(EGA)  350(VGA)

3=          480(VGA).

Note: Set to all zero on a hardware reset.

Note: On the VGA this register can be read from port 3CCh.

3C3h (W): Video Subsystem Enable Register

bit 0  Enables the VGA display if set

3C4h index  0  (r/W): Sequencer: Reset

bit 0  (EGA) Asynchronous Reset if clear

0  (VGA) Synchronous Reset just as bit 1

1  Synchronous Reset if clear

3C4h index  1  (r/W): Sequencer: Clocking Mode

bit 0  If set character clocks are 8 dots wide, else 9.

1  (EGA Only) If set the CRTC uses 2/5 of the clock cycles, else 4/5.

2  If set loads video serializers every other character

clock cycle, else every one.

3  If set the Dot Clock is Master Clock/2, else same as

Master Clock (See 3C2h bit 2-3). (Doubles pixels).

4  (VGA Only) If set loads video serializers every fourth

character clock cycle, else every one.

5  (VGA Only) if set turns off screen and gives all memory

cycles to the CPU interface.

3C4h index  2  (r/W): Sequencer: Map Mask Register

bit 0  Enable writes to plane 0 if set

1  Enable writes to plane 1 if set

2  Enable writes to plane 2 if set

3  Enable writes to plane 3 if set

3C4h index  3  (r/W): Sequencer: Character Map Select Register

bit 0-1  (EGA) Selects EGA Character Map (0..3) if bit 3 of

the character attribute is clear.

2-3  (EGA) Selects EGA Character Map (0..3) if bit 3 of

the character attribute is set.

0,1,4  (VGA) Selects VGA Character Map (0..7) if bit 3 of

the character attribute is clear.

2,3,5  (VGA) Selects VGA Character Map (0..7) if bit 3 of

the character attribute is set.

Character Maps are placed at:

Map no. (EGA/VGA)       Map no. (VGA)

0       0k              4      8k

1      16k              5     24k

2      32k              6     40k

3      48k              7     56k

3C4h index  4  (r/W): Sequencer: Memory Mode Register

bit 0  Set if in an alphanumeric mode, clear in graphics modes.

1  Set if more than 64kbytes on the adapter.

2  Enables Odd/Even addressing mode if set.

Odd/Even mode places all odd bytes in plane 1&3, and

all even bytes in plane 0&2.

3  (VGA Only)  If set address bit 0-1 selects video memory

planes (256 color mode), rather than the

Map Mask and Read Map Select Registers.

3C6h (R/W): PEL Mask          (VGA Only)

bit 0-7  Reserved for internal use.

Should always be set to FFh.

3C7h (R): DAC State Register  (VGA Only)

bit 0-1  0 indicates the DAC is in Read Mode and 3 indicates

write mode.

3C7h (W): PEL Address Read Mode           (VGA Only)

bit 0-7  The PEL data register (0..255) to be read from 3C9h.

Note:  After reading the 3 bytes at 3C9h this register will

increment, pointing to the next data register.

3C8h (R/W): PEL Address Write Mode        (VGA Only)

bit 0-7  The PEL data register (0..255) to be written to 3C9h.

Note:  After writing the 3 bytes at 3C9h this register will

increment, pointing to the next data register.

3C9h (R/W): PEL Data Register             (VGA Only)

bit 0-5  Color value

Note:  Each read or write of this register will cycle through first

the registers for Red, Blue and Green, then increment the

appropriate address register, thus the entire palette can be

loaded by writing 0 to the PEL Address Write Mode register 3C8h

and then writing all 768 bytes of the palette to this register.

3CAh (R): Feature Control Register           (VGA Only)

Bit 3  (VGA Only) Vertical Sync Select

If set Vertical Sync to the monitor is the logical OR

of the vertical sync and the vertical display enable.

Note: This register is written to port 3dAh and read from 3CAh.

3CAh (W): Graphics 2 Position                (EGA Only)

bit 0-1  Select which bit planes should be controlled by

Graphics Controller #2. Always set to 1.

3CCh (R): Miscellaneous Output Register      (VGA Only)

bit 0  If set Color Emulation. Base Address=3Dxh

else Mono Emulation. Base Address=3Bxh.

1  Enable CPU Access to video memory if set

2-3  Clock Select

0= 25MHz, 1= 28MHz, 2= Reserved

5  When in Odd/Even modes Select High 64k bank if set

6  Horizontal Sync Polarity. Negative if set

7  Vertical Sync Polarity. Negative if set

Bit 6-7 indicates the number of lines on the display:

0=Reserved, 1=400, 2=350, 3=480.

Note: This register is written to port 3C2h and read from port 3CCh.

3CCh (W): Graphics 1 Position                (EGA Only)

bit 0-1  Select which bit planes should be controlled by

Graphics Controller #1. Always set to 0.

3CEh index  0  (r/W): Graphics: Set/Reset Register

bit 0  If in Write Mode 0 and bit 0 of 3CEh index 1 is set

a write to display memory will set all the bits in

plane 0 of the byte to this bit, if the corresponding

bit is set in the Map Mask Register (3CEh index 8).

1  Same for plane 1 and bit 1 of 3CEh index 1.

2  Same for plane 2 and bit 2 of 3CEh index 1.

3  Same for plane 3 and bit 3 of 3CEh index 1.

3CEh index  1  (r/W): Graphics: Enable Set/Reset Register

bit 0  If set enables Set/reset of plane 0 in Write Mode 0.

1  Same for plane 1.

2  Same for plane 2.

3  Same for plane 3.

3CEh index  2  (r/W): Graphics: Color Compare Register

bit 0-3  In Read Mode 1 each pixel at the address of the byte read

is compared to this color and the corresponding bit in

the output set to 1 if they match, 0 if not.

The Color Dont Care Register (3CEh index 7) can exclude

bitplanes from the comparison.

3CEh index  3  (r/W): Graphics: Data Rotate

bit 0-3  Number of positions to rotate data right before it is

written to display memory.

4-5  In Write Mode 2 this field controls the relation between

the data written from the CPU, the data latched from the

previous read and the data written to display memory:

0: CPU Data is written unmodified

1: CPU data is ANDed with the latched data

2: CPU data is ORed  with the latch data.

3: CPU data is XORed with the latched data.

3CEh index  4  (r/W): Graphics: Read Map Select Register

bit 0-1  Number of the plane read Mode 0 will read from.

3CEh index  5  (r/W): Graphics: Mode Register

bit 0-1  Write Mode: Controls how data from the CPU is

transformed before being written to display memory:

0: Mode 0 works as a Read-Modify-Write operation.

First a read access loads the data latches of the EGA/VGA

with the value in video memory at the addressed location.

Then a write access will provide the destination address

and the CPU data byte. The data written is modified by the

function code in the Data Rotate register (3CEh index 3) as

a function of the CPU data and the latches, then data

is rotated as specified by the same register.

1: Mode 1 is used for video to video transfers.

A read access will load the data latches with the contents

of the addressed byte of video memory. A write access will

write the contents of the latches to the addressed byte.

Thus a single MOVSB instruction can copy all pixels in the

source address byte to the destination address.

2: Mode 2 writes a color to all pixels in the addressed byte

of video memory. Bit 0 of the CPU data is written to plane 0

etc. Individual bits can be enabled or disabled through the

Bit Mask register (3CEh index 8).

3: (VGA Only)

2  (EGA Only) Forces all outputs to a high impedance state if set.

3  Read Mode

0: Data is read from one of 4 bit planes depending

on the Read Map Select Register (3CEh index 4).

1: Data returned is a comparison between the 8 pixels

occupying the read byte and the color in the

Color Compare Register (3CEh index 2).

A bit is set if the color of the corresponding

pixel matches the register.

4  Enables Odd/Even mode if set (See 3C4h index 4 bit 2).

5  Enables CGA style 4 color pixels using even/odd bit pairs

if set.

6  (VGA Only) Enables 256 color mode if set.

3CEh index  6  (r/W): Graphics: Miscellaneous Register

bit 0  Indicates Graphics Mode if set, Alphanumeric mode else.

1  Enables Odd/Even mode if set.

2-3  Memory Mapping:

0: use A000h-BFFFh

1: use A000h-AFFFh   EGA/VGA Graphics modes

2: use B000h-B7FFh   Monochrome modes

3: use B800h-BFFFh   CGA modes

3CEh index  7  (r/W): Graphics: Color Dont Care Register

bit 0  Ignore bit plane 0 in Read mode 1 if clear.

1  Ignore bit plane 1 in Read mode 1 if clear.

2  Ignore bit plane 2 in Read mode 1 if clear.

3  Ignore bit plane 3 in Read mode 1 if clear.

3CEh index  8  (r/W): Graphics: Bit Mask Register

bit 0-7  Each bit if set enables writing to the corresponding

bit of a byte in display memory.

3d4h index  0  (r/W): CRTC: Horizontal Total Register

bit 0-7  (EGA) Horizontal Total Character Clocks-2

0-7  (VGA) Horizontal Total Character Clocks-5

3d4h index  1  (r/W): CRTC: Horizontal Display End Register

bit 0-7  Number of Character Clocks Displayed -1

3d4h index  2  (r/W): CRTC: Start Horizontal Blanking Register

bit 0-7  The count at which Horizontal Blanking starts

3d4h index  3  (r/W): CRTC: End Horizontal Blanking Register

bit 0-4  Horizontal Blanking ends when the last 5 (6 for VGA)

bits of the character counter equals this field.

(VGA) The sixth bit is found in port 3d4h index 5 bit 7.

5-6  Number of character clocks to delay start of display

after Horizontal Total has been reached.

7  (VGA Only) Access to Vertical Retrace registers if set

3d4h index  4  (r/W): CRTC: Start Horizontal Retrace Register

bit 0-7  Horizontal Retrace starts when the Character Counter

reaches this value.

3d4h index  5  (r/W): CRTC: End Horizontal Retrace Register

bit 0-4  Horizontal Retrace ends when the last 5 bits of the

character counter equals this value.

5-6  Number of character clocks to delay start of display

after Horizontal Retrace.

7  (EGA) Provides Smooth Scrolling in Odd/Even mode.

When set display starts from an odd byte.

7  (VGA) bit 5 of the End Horizontal Blanking count

(See 3d4h index 3 bit 0-4).

3d4h index  6  (r/W): CRTC: Vertical Total Register

bit 0-7  Lower 8 bits of the Vertical Total

Bit 8 is found in 3d4h index 7 bit 0.

(VGA) Bit 9 is found in 3d4h index 7 bit 5.

Note: For the VGA this value is the number of scan lines in the display -2.

3d4h index  7  (r/W): CRTC: Overflow Register

bit 0  Bit 8 of Vertical Total (3d4h index 6)

1  Bit 8 of Vertical Display End (3d4h index 12h)

2  Bit 8 of Vertical Retrace Start (3d4h index 10h)

3  Bit 8 of Start Vertical Blanking (3d4h index 15h)

4  Bit 8 of Line Compare Register (3d4h index 18h)

5  (VGA) Bit 9 of Vertical Total (3d4h index 6)

6  (VGA) Bit 9 of Vertical Display End (3d4h index 12h)

7  (VGA) Bit 9 of Vertical Retrace Start (3d4h index 10h)

3d4h index  8  (r/W): CRTC: Preset Row Scan Register

bit 0-4  Number of lines we have scrolled down in the first

character row. Provides Smooth Vertical Scrolling.

5-6  (VGA Only) Number of bytes to skip at the start of

scanline. Provides Smooth Horizontal Scrolling

together with the Horizontal Panning Register

(3C0h index 13h).

3d4h index  9  (r/W): CRTC: Maximum Scan Line Register

bit 0-4  Number of scan lines in a character row -1

5  (VGA) Bit 9 of Start Vertical Blanking

6  (VGA) Bit 9 of Line Compare Register

7  (VGA) Doubles each scan line if set.

I.e displays 200 lines on a 400 display.

3d4h index  Ah (r/W): CRTC: Cursor Start Register

bit 0-4  First scanline of cursor within character.

5  (VGA) Turns Cursor off if set

3d4h index  Bh (r/W): CRTC: Cursor End Register

bit 0-4  Last scanline of cursor within character

5-6  Delay of cursor data in character clocks.

3d4h index  Ch (r/W): CRTC: Start Address High Register

bit 0-7  Upper 8 bits of the start address of the display buffer

3d4h index  Dh (r/W): CRTC: Start Address Low Register

bit 0-7  Lower 8 bits of the start address of the display buffer

3d4h index  Eh (r/W): CRTC: Cursor Location High Register

bit 0-7  Upper 8 bits of the address of the cursor

3d4h index  Fh (r/W): CRTC: Cursor Location Low Register

bit 0-7  Lower 8 bits of the address of the cursor

3d4h index 10h (R): CRTC: Light Pen High Register     (EGA Only)

bit 0-7  (EGA Only)  Upper 8 bits of the address of the

lightpen position.

3d4h index 10h (r/W): CRTC: Vertical Retrace Start Register

bit 0-7  Lower 8 bits of Vertical Retrace Start. Vertical Retrace

starts when the line counter reaches this value.

Bit 8 is found in 3d4h index 7 bit 2.

(VGA Only) Bit 9 is found in 3d4h index 7 bit 7.

3d4h index 11h (R): CRTC: Light Pen Low Register      (EGA Only)

bit 0-7  (EGA Only)  Lower 8 bits of the address of the

lightpen position.

3d4h index 11h (r/W): CRTC: Vertical Retrace End Register

bit 0-3  Vertical Retrace ends when the last 4 bits of the

line counter equals this value.

4  if clear Clears pending Vertical Interrupts.

5  Vertical Interrupts (IRQ 2) disabled if set.

Can usually be left disabled, but some systems

(including PS/2) require it to be enabled.

6  (VGA Only) If set selects 5 refresh cycles per

scanline rather than 3.

7  (VGA Only) Disables writing to registers 0-7 if set

3d4h index 12h (r/W): CRTC: Vertical Display End Register

bit 0-7  Lower 8 bits of Vertical Display End. The display

ends when the line counter reaches this value.

Bit 8 is found in 3d4h index 7 bit 1.

(VGA Only) Bit 9 is found in 3d4h index 6.

3d4h index 13h (r/W): CRTC: Offset register

bit 0-7  Number of bytes in a scanline / K. Where K is 2 for

byte mode, 4 for word mode and 8 for Double Word mode.

3d4h index 14h (r/W): CRTC: Underline Location Register

bit 0-4  Position of underline within Character cell.

5  (VGA Only) If set memory address is only changed

every fourth character clock.

6  (VGA Only) Double Word mode addressing if set

3d4h index 15h (r/W): CRTC: Start Vertical Blank Register

bit 0-7  Lower 8 bits of Vertical Blank Start. Vertical blanking

starts when the line counter reaches this value.

Bit 8 is found in 3d4h index 7 bit 3.

3d4h index 16h (r/W): CRTC: End Vertical Blank Register

bit 0-4  (EGA) Vertical blanking stops when the lower 5 bits

of the line counter equals this field.

0-6  (VGA) Vertical blanking stops when the lower 7 bits

of the line counter equals this field.

3d4h index 17h (r/W): CRTC: Mode Control Register

bit 0  If clear use CGA compatible memory addressing system

by swapping address bit 0 and 13.

Creates 2 banks for even and odd scan lines.

1  If clear use Hercules compatible memory addressing

system by swapping address bit 1 and 14.

2  If set increase scan line counter only every second line.

3  If set increase memory address counter only every other

character clock.

4  (EGA Only) If set disable the EGA output drivers. This bit

is used for other purposes in some Super VGA chips.

5  When in Word Mode bit 15 is rotated to bit 0 if this bit

is set else bit 13 is rotated into bit 0.

6  If clear system is in word mode. Addresses are rotated

1 position up bringing either bit 13 or 15 into bit 0.

7  Clearing this bit will reset the display system

until the bit is set again.

3d4h index 18h (r/W): CRTC: Line Compare Register

bit 0-7  Lower 8 bits of the Line Compare. When the Line counter

reaches this value, the display address wraps to 0.

Privides Split Screen facilities.

Bit 8 is found in 3d4h index 7 bit 4.

(VGA Only) Bit 9 is found in 3d4h index 9 bit 6.

3dAh (R): Input Status #1 Register

bit 0  Either Vertical or Horizontal Retrace active if set

1  (EGA Only) Light Pen has triggered if set

2  (EGA Only) Light Pen switch is open if set

3  Vertical Retrace in progress if set

4-5  (EGA Only) Shows two of the 6 color outputs,

depending on 3C0h index 12h bit 4-5:

Attr: Bit 4-5:   Out bit 4  Out bit 5

0          Blue       Red

1        I Blue       Green

2        I Red      I Green

3dAh (W): Feature Control Register

bit 0  (EGA Only) Output to pin 21 of the Feature Connector.

1  (EGA Only) Output to pin 20 of the Feature Connector.

3  (VGA Only) Vertical Sync Select

If set Vertical Sync to the monitor is the logical OR

of the vertical sync and the vertical display enable.

Note: On the VGA this register can be read from port 3CAh.