MIDPAK

written by

John W. Ratcliff

Copyright (c) 1994

The Audio Solution

The MIDI driver MIDPAK hooks into user interrupt vector 66h to provide a

clean clear programmers interface. This document describes the two ways

you can access the midi sound package.	The first is the interrupt

level interface.  The second is a link library interface that provides a

set of C or assembly language callable functions to access not only the

digitized sound drivers but also provides usefull support routines.

These glue code functions are located in the source file MIDPAK.ASM.

MIDPAK uses the same interupt vector as DIGPAK.  DIGPAK describes the

complete set of Digitized sound drivers provided by THE Audio Solution.

MIDPAK is fully compatible with DIGPAK.  If your application needs to

play back both MIDI music and digitized sound you simply first load the

digitized sound driver you need, and then load the MIDPAK midi driver on

top of it.  The MIDPAK driver detects the presence of the DIGPAK sound

driver and re-routes all calls through it.  If the digitized sound

hardware does not support independent playback of digitized sound (DMA

support: SoundBlaster, and ProAudio Spectrum do.) then MIDI music

playback will shut down while the digitized sound effect is playing.

MIDI music playback will continue once the digitized sound effect has

completed.  This is completely transparent to the running application.

MIDPAK uses the set of MIDI sound drivers developed by Miles Design

Incorporated.  These drivers vary in size and can be noted by the

extension of .ADV.  MIDPAK will always load the sound driver MUSIC.ADV

when started.  The application should copy the apropriate sound driver

over as MUSIC.ADV before loading MIDPAK.

MIDPAK does not play .MID files directly.  You must convert the MIDI

file (with the extension of .MID) into an eXtended MIDI file (.XMI)

by using the program MIDIFORM or the MENU utility.  The Miles

eXtended MIDI drivers support MIDI channels 2-9 melodic and channel 10

for percussions.

***********************************************************************

*************** INT 66h Interface Specification ***********************

***********************************************************************

Function #1: UnloadMidPak, removes the MIDPAK tsr from memory, NOT TO BE

USED by APPLICATION software!

INPUT:	AX = 700h	Command number.

OUTPUT: 		None.

Function #2: DigPakAvailable, determine whether or not a digpak sound driver

as available, underneath the MIDPAK driver.

INPUT: AX = 701h	Command number.

OUTPUT:AX = 0		Digpak is not available.

AX = 1		Digpak is available.

Function #3: PlaySequence, plays a sequence from the currently registered

xmidi file.

INPUT: AX = 702h	Command number.

BX = SEQ 	Sequence number, numbered starting from zero.

OUTPUT:AX = 1		Sequence is being played.

AX = 0		Sequence not available.

Function #4: SegueSequence, register a new sequence to be segueded in at the

next trigger event, with the activation

code specified.  If the activation code passed is

a -1, then this sequence will be changed to at the

next trigger.

INPUT:	AX = 703h	Command number.

BX = SEQ	Sequence number to segue to.

CX = ACT	Activation event code, -1 means next trigger.

Function #5, RegisterXmidi, Register an XMIDI file by address for playback.

INPUT:	AX = 704h	Command number.

BX = Offset	Offset portion of far address of XMIDI data.

CX = Segment	Segment portion of far address of XMIDI data.

SI = Low len	Low word of length of XMIDI data.

DI = High len	High word of length of XMIDI data.

OUTPUT: AX = 0		Unable to register XMIDI data.

AX = 1		XMIDI file registered resident.  This means

that the XMIDI file was able to be held

entirely in MIDPAK's internal buffer area.

The application can throw away the memory

associated with this XMIDI file because

MIDPAK has made a copy of it for itself.

This is very useful in virtual memory

environments where the application program

does not always have fixed addresses in

memory.  It also allows MIDPAK to play

back MIDI files in the background from DOS.

AX = 2		XMIDI file was registered to the appilcation.

The caller is responsible for making sure that

this fixed memory address is always contains

the data as passed.

Function #6:	MidiStop, stop playing current MIDI sequence.

INPUT	AX = 705h	Command.

OUTPUT: None.

Function #7:	Unsed

Function #8:	ReportTriggerCount -> returns trigger event counter,

and last event code.

INPUT:	AX = 707h	Command number.

OUTPUT: AX = COUNT	Count of # of callbacks, since last reset.

DX = ID 	Event ID of last callback.

See callback trigger in XMIDI spec.

Function #9:	ResetTriggerCount -> reset's the trigger event counter to zero.

INPUT:	AX = 708h	Command number.

OUTPUT: None.

Function #10:	OBSOLETE FUNCTION.

Function #11:	OBSOLETE FUNCTION

Function #12:	ResumePlaying -> resume playing stopped sequence.

INPUT:	AX = 70Bh	Command number.

Function #13:	SequenceStatus -> report sequence status.

INPUT:	AX = 70Ch	Command number.

OUTPUT: AX = Status.

SEQ_STOPPED 0		// equates for SequenceStatus()

SEQ_PLAYING 1		// Sequence is currently playing.

SEQ_DONE 2		// A sequence is DONE playing.

Function #14:	RegisterXmidiFile( register by filename)

INPUT:	AX = 70Dh	Command number.

BX = Offset	Offset portion of filename address.

CX = Segment	Segment portion of filename address.

Function #15:	RelativeVolume->

INPUT:	AX = 70Eh	Command number.

OUTPUT: AX = VOL	Current relative volume.

Function #16:	SetRelativeVolume->

INPUT:	AX = 70Fh	Command number.

Function #17:	BootstrapMidPak

INPUT:	AX = 710h	Command number.

BX:CX -> segment,offset of ADV driver.

DX:SI -> segment,offset of AD file.

Function #18: PollMidPak

INPUT:	AX = 711h

This function is used in conjunction with PMIDPAK.COM.	PMIDPAK is

the polled version of MIDPAK.  Normal MIDPAK steals either the timer

interrupt or the real-time clock interrupt and services it as a rate

of 120 hertz.  However, some more advanced application want to service

the hardware interrupts themselves, may want to play back the music

at a different quantization rate, or may want to sync the service

rate to graphics routines, to avoid any potential of their graphics

routines being interrupted by the music.

After MIDPAK is installed it restores the timer interrupt vector and

is aquiecent until the application program invokes int 66h with the

command 0711h.	You should invoke this interrupt at 120 hertz or

to the quantization rate you used when running MIDIFORM.  When

running MIDIFORM you can specify a quantization rate for your music

other than the default of 120 hertz.  As you lower the quantization

rate you will here some degradation in the quality of the music

since ASDR will not be responding as quickly.  Reducing it to say

60 hertz won't make much of a noticable difference, but reducing it

to 15 to 30 hertz will cause noticable degradation.  Obviously running

many channels of MIDI music out of an FM synthesis device like an

Adlib is going to take some CPU.  By re-quantiziing your music and

servicing MIDPAK at the rate you are comforatble with, you can

balance machine resources nicely yourself.

Function #19: MidpakClock

INPUT:	AX = 712h

OUTPUT: AX:DX Current heartbeat count since MidPak startup.

120 hertz heart beat, application can use this for

timing.

BX:CX -> forms the far address pointing to this

heartbeat counter.

BX-> is the offset portion.

CX-> is the segment portion.

This function reports the heartbeat counter for MidPak.  Starting

at zero since MidPak startup, this value is the long word result of

the heartbeat counter for MidPak.  A 120 hertz timer that your

application software can use for timing (in a polled fashion).

Function #20: TriggerCountAddress

INPUT:	AX = 713h

OUTPUT: AX:DX form for address to point to the integer TriggerCount.

This is the counter that gets incremented each time

a Controller 119 is encountered in the midi file.

Function #21: EventIDAddress

INPUT:	AX = 714h

OUTPUT: AX:DX form an address to point to the integer EventID,

with is the EventID of the last controller 119

trigger event encountered.

Function #22: Undocumented

Function #23: ReportSequenceNumber

INPUT:	AX = 716h

OUTPUT: Report currently playing sequence number.

extern short	 cdecl CheckMidiIn(void); // Returns 1 if MIDPAK is installed, 0 if not.

extern short	 cdecl DigPakAvailable(void);	  // Returns 1 if DIGPAK is installed, 0 if not.

/****************************************************************************/

/* These flags are returned by the XMIDI registration call.		      */

/****************************************************************************/

#define FAILURE_TO_REGISTER 0	 // Xmidi file registration failed.

#define REGISTERED_RESIDENT 1   // Resident driver holds midi file now.

// The application can throw away the memory

// if it wants to.

#define REGISTERED_APPLICATION 2  // Driver didn't have a big enough buffer

// area reserved to hold the audio data

// so the application is responsible for

// keeping the memory for this sequence

// while it is registered.

extern short	 cdecl PlaySequence(short seqnum); // Play a particular sequence number from

// the currently registered xmidi file.

#define NEXT_CALLBACK -1	 // Activation on next callback.

extern short	 cdecl SegueSequence(short seqnum,short activate);

// Switch sequence to this sequence when next

// callback trigger event is hit with the

// event number equal to activate.	     If activate

// is set to -1 then the next event causes the

// segue to occur.

extern short	 cdecl RegisterXmidi(char	  *xmidi,long int size);

// Registers an extended midi file for playback.

// This call will register all sequences.

extern short	 cdecl MidiStop(void);	  // Stop playing current sequence.

extern long int  cdecl ReportCallbackTrigger(void); // Low word is trigger count.

// High word is last event ID.

extern void  cdecl ResetCallbackCounter(void);	 // Reset callback counter to zero.

extern void  cdecl ResumePlaying(void); 	 // Resume playing last sequence.

#define SEQ_STOPPED 0		  // equates for SequenceStatus()

#define SEQ_PLAYING 1

#define SEQ_DONE 2

extern short	  cdecl SequenceStatus(void);	  // Report current sequence play status.

extern short	  cdecl RelativeVolume(short vol); // Report current volume.

extern void  cdecl SetRelativeVolume(short vol,short time); // Set volume, over time period.

#define NOBUFFER 1		  // No resident buffer available.

#define FILENOTFOUND 2		  // The file was not found.

#define FILETOBIG 3		  // The file exceeds the reserved buffer size.

#define REGISTRATIONERROR 4	 // Error registering the XMI file.

extern short	  cdecl RegisterXmidiFile(char *fname); // Register by filename.

extern void  cdecl PollMidPak(void); // Poll MidPak for music processing.

extern long int  cdecl MidPakClock(void); // Return MIDPAK heartbeat count (120hz)

extern long int  * cdecl MidPakClockAddress(void); // Return address of midpak clock.

extern short	 *  cdecl TriggerCountAddress(void); // Report address of trigger count.

extern short	 *  cdecl EventIDAddress(void); // Report address of event id.

extern short	 cdecl ReportSequenceNumber(void);

extern short	 cdecl InitMP(char *midpak,char *adv,char	  *ad); // Init MIDI driver.

extern void cdecl DeInitMP(char *midpak); // Unload a previously loaded sound driver.

***************************************************************************

Written by John W. Ratcliff (c) 1994

Compuserve: 70253,3237

Genie: J.RATCLIFF3

BBS: 1-314-939-0200

Addresss:

747 Napa Lane

St. Charles, MO 63304

A $500 per product license fee applies to all commercial software

products distributed with ANY MIDPAK drivers.

To pay a license, simply write a check for $500 payable to The Audio

Solution, 747 Napa Lane, St. Charles, MO 63304 with a copy of your

commerical product.  You will receive a signed license agreement from

The Audio Solution shortly thereafter. This license fee applies

specifically to the inclusion with your distribution disk any of the

MIDPAK drivers from The Audio Solution These drivers are copyrighted

works, created by me, to enhance the use of sound and music in DOS based

commercial software.  The license fees collected are used to maintain

the drivers and keep the BBS running.  There is a seperate license fee

for the use and distribution of MIDPAK drivers.

See accompaning documentation regarding license fees for DIGPAK

distribution.  You would be ill-advised to distribute a commercial

product containing either DIGPAK and/or MIDPAK drivers without having

paid the distribution license fee.  Since your product would contain

unlicensed copyrighted software from The Audio Solution, your product

could be required to be immediately removed from retail distribution.  I

doubt this is going to be a problem.  Clearly if your product is

enhanced by the use of these drivers, your company can easily afford a

nominal license fee of $500 in exchange for getting the use of several

man-years of software engineering resources.